# < Robot !

###### R-30+B Plus/R-30+B Mate Plus/R-30+B Compact Plus

###### CONTROLLER

Remote Motion Interface

OPERATOR'S MANUAL

#### B-84184EN/03

© FANUC CORPORATION, 2020

Translation of the original instruction

The original language of this manual is English. The other language manuals are a translation from the English one.

Thank you very much for purchasing FANUC Robot.

Before using the Robot, be sure to read the "FANUC Robot series SAFETY HANDBOOK (B-80687EN)" and understand the content.

- No part of this manual may be reproduced in any form.

- All specifications and designs are subject to change without notice.

The products in this manual are controlled based on Japan’s “Foreign Exchange and Foreign Trade Law”. The export from Japan may be subject to an export license by the government of Japan.

Further, re-export to another country may be subject to the license of the government of the country from where the product is re-exported. Furthermore, the product may also be controlled by re-export regulations of the United States government.

Should you wish to export or re-export these products, please contact FANUC for advice.

In this manual, we endeavor to include all pertinent matters. There are, however, a very large number of operations that must not or cannot be performed, and if the manual contained them all, it would be enormous in volume. It is, therefore, requested to assume that any operations that are not explicitly described as being possible are "not possible".

B-84184EN/03	SAFETY PRECAUTIONS

SAFETY PRECAUTIONS

This chapter describes the precautions which must be followed to enable the safe use of the robot. Before using the robot, be sure to read this chapter thoroughly.

For detailed functions of the robot operation, read the relevant operator's manual to understand fully its specification.

For the safety of the operator and the system, follow all safety precautions when operating a robot and its peripheral equipment installed in a work cell. For safe use of FANUC robots, you must read and follow the instructions in the FANUC Robot series SAFETY HANDBOOK (B-80687EN).

PERSONNEL

Personnel can be classified as follows:

Operator:

- Turns the robot controller power ON/OFF

- Starts the robot program from the operator panel Programmer or Teaching operator:

- Operates the robot

- Teaches the robot inside the safeguarded space Maintenance technician:

- Operates the robot

- Teaches the robot inside the safeguarded space

- Performs maintenance (repair, adjustment, replacement)

  - The operator is not allowed to work in the safeguarded space.

  - The programmer or teaching operator and maintenance technician are allowed to work in the safeguarded space. Work carried out in the safeguarded space include transportation, installation, teaching, adjustment, and maintenance.

  - Based on the Ordinance on Industrial Safety and Health in Japan or laws in each country, to work inside the robot set safeguarded space, the following is required.

    - Perform risk assessment for the entire robot system.

    - Based on the risk assessment result, all personnel must take appropriate professional safety training courses.

  - Table s-1 lists the work outside the safeguarded space. In this table, the symbol “￮” means the work is allowed to be carried out by the specified personnel.

Table s-1 Work Performed Outside the Safeguarded Space

|  | Operator | Programmer or Teaching Operator | Maintenance Technician |
| --- | --- | --- | --- |
| Turn power ON/OFF to Robot controller | o | o | o |
| Select operating mode (AUTO, T1, T2) |  | o | o |
| Select remote/local mode |  | o | o |
| Select robot program with teach pendant |  | o | o |

SAFETY PRECAUTIONS	B-84184EN/03

|  | Operator | Programmer or Teaching Operator | Maintenance Technician |
| --- | --- | --- | --- |
| Select robot program with external device |  | o | o |
| Start robot program with operator’s panel | o | o | o |
| Start robot program with teach pendant |  | o | o |
| Reset alarm with operator’s panel |  | o | o |
| Reset alarm with teach pendant |  | o | o |
| Set data on teach pendant |  | o | o |
| Teaching with teach pendant |  | o | o |
| Emergency stop with operator’s panel | o | o | o |
| Emergency stop with teach pendant | o | o | o |
| Operator panel maintenance |  |  | o |
| Teach pendant maintenance |  |  | o |

During robot operation, programming and maintenance, the operator, programmer, teaching operator and maintenance engineer take care of their own safety using at least the following safety protectors:

- Use clothes, uniform, overall adequate for the work

- Safety shoes

- Helmet

DEFINITION OF SAFETY NOTATIONS

To ensure the safety of users and prevent damage to the machine, this manual indicates each precaution on safety with "WARNING" or "CAUTION" according to its severity. Supplementary information is indicated by "NOTE". Read the contents of each "WARNING", "CAUTION" and "NOTE" before using the robot.

| Symbol | Definitions |
| --- | --- |
| WARNING | Used if hazard resulting in the death or serious injury of the user will be expected to occur if he or she fails to follow the approved procedure. |
| CAUTION | Used if a hazard resulting in the minor or moderate injury of the user, or equipment damage may be expected to occur if he or she fails to follow the approved procedure. |
| NOTE | Used if a supplementary explanation not related to any of WARNING and CAUTION is to be indicated. |

B-84184EN/03	TABLE OF CONTENTS

TABLE OF CONTENTS

##### SAFETY PRECAUTIONS	s-1

- INTRODUCTION	1

  - REMOTE MOTION INTERFACE OVERVIEW	1

  - HARDWARE AND SOFTWARE REQUIREMENTS	1

  - COMPATIBILITY	2

  - LIMITATIONS	2

    - Number of Motion Groups Supported	2

    - Hot Start is Disabled	2

    - Original Path Resume Function is Disabled	2

    - Instruction Packet Buffer	2

    - Minimum Motion Duration	3

    - Other Available Communication Options	3

- REMOTE MOTION APPLICATION PROGRAM INTERFACE	5

  - REMOTE MOTION APPLICATION PROGRAM INTERFACE OVERVIEW	5

  - COMMUNICATION PACKETS	5

    - Packet Exchange	6

    - Controller Disconnect Packet	7

    - Timeout Terminate Packet	8

    - System Fault Packet	8

  - ROBOT COMMAND PACKETS	8

    - Packet To Initialize Remote Motion	9

    - Packet To Abort Remote Motion TP Program	11

    - Packet To Pause Remote Motion Program	11

    - Packet To Continue Remote Motion Program	11

    - Packet To Read Controller Error	12

    - Packet To Set The Current UFrame-UTool Number	13

    - Packet To Get Controller Status	14

    - Packet To Read User Frame Data	15

    - Packet To Set User Frame Data	15

    - Packet To Read User Tool Data	16

    - Packet To Set User Tool Data	17

    - Packet To Read Input Port	17

    - Packet To Write Digital Output Port	18

    - Packet To Read Current Robot Cartesian Position	18

    - Packet To Read Current Robot Joint Angles	19

    - Packet To Set Speed Override	20

    - Packet To Get Current User/Tool Frame Number	20

    - Packet To Read Position Register Data	21

    - Packet To Write Position Register Data	22

    - Packet To Reset Robot Controller	22

    - Packet To Read Current Tool Center Point Speed	23

  - TEACH PENDANT PROGRAM INSTRUCTION PACKETS	23

    - Packet To Wait for DIN Instruction	25

    - Packet To Set User Frame Instruction	26

    - Packet To Set User Tool Instruction	26

    - Packet To Add Wait Time Instruction	26

    - Packet To Set Payload Instruction	27

TABLE OF CONTENTS	B-84184EN/03

    - Packet To Call A Program	27

    - Packet To Add Linear Motion Instruction	28

      - Packet to add two groups linear motion instruction	32

    - Packet To Add Linear Incremental Motion Instruction	36

      - Packet to add two groups linear incremental motion instruction	37

    - Packet To Add Joint Motion Instruction	38

      - Packet to add two group joint motion instruction	40

    - Packet To Add Joint Incremental Motion Instruction	42

      - Packet to add two groups joint incremental motion instruction	43

    - Packet To Add Circular Motion Instruction	44

      - Packet to add two groups circular motion instruction	46

    - Packet To Add Circular Incremental Motion Instruction	48

      - Packet to add two groups circular incremental motion instruction	49

    - Packet To Add Joint Motion With Joint Representation	50

      - Packet to add two groups joint motion with joint representation instruction	51

    - Packet To Add Joint Incremental Motion With Joint Representation	52

      - Packet to add two groups joint incremental motion with joint representation instruction	53

    - Packet To Add Linear Motion With Joint Representation	53

      - Packet to add two groups linear motion with joint representation instruction	55

    - Packet To Add Linear Incremental Motion With Joint Representation	56

      - Packet to add two groups linear incremental motion with joint representation instruction. 57

    - Packet To Add Spline Motion Instruction	57

      - Packet to add two groups spline motion instruction	60

    - Packet To Add Spline Motion With Joint Representation	61

      - Packet to add two groups Spline motion with joint representation instruction	63

    - Unknown Packet Handling	63

  - POSITION RECORDING	64

    - Using Position Registers	64

    - Using RMI Position Record Menu	64

#### REMOTE DEVICE CONTROL FLOW	69

  - OVERVIEW OF REMOTE DEVICE CONTROL FLOW	69

  - INSTRUCTION BUFFER HANDLING	71

  - HOLD STATE ERROR HANDLING	71

  - JUMP AND SKIP INSTRUCTIONS	71

  - ERROR HANDLING	71

  - VISION PROCESS	72

#### DATA LOGGING	73

#### ASCII STRING INSTRUCTION PACKETS	75

  - ASCII STRING PACKET HANDSHAKING	75

  - SYSTEM VARIABLE SETTING	76

  - ASCII STRING INSTRUCTION PACKET	77

    - ASCII String Packet For Single Group Controller	77

    - ASCII String Packet For Two Group Motion Instruction	81

    - Communication Packet	81

    - Modified FRC_Initialize Command	81

  - LIST OF INVALID ASCII STRING INSTRUCTIONS	83

B-84184EN/03	TABLE OF CONTENTS

## APPENDIX

#### A APPENDIX	87

  - RMI ERRORID REFERENCE TABLE	87

TABLE OF CONTENTS	B-84184EN/03

B-84184EN/03	1 INTRODUCTION

# INTRODUCTION

## REMOTE MOTION INTERFACE OVERVIEW

Remote Motion Interface (RMI) allows a remote device to control the robot motion by sending teach pendant program instructions to the controller. The controller creates a special teach pendant (TP) program, executes this program and appends these new instructions from the remote device to the TP program on the fly.

Figure 1.1 System Overview

This option's main target is the material handling market with simple pick and place operations. Due to a limited number of instructions available for the Remote Motion Interface, it is not meant for complicated applications such as arc welding that reads real time sensor feedback and dynamically adjusts the robot path.

## HARDWARE AND SOFTWARE REQUIREMENTS

Hardware

The Remote Motion Interface option requires an R-30iB Plus controller.

Software

The controller needs to be loaded with the Remote Motion Interface (R912) option.

Communication

Ethernet connection.

1 INTRODUCTION	B-84184EN/03

## COMPATIBILITY

The Remote Motion Interface (R912) option is not compatible with the PLC Motion Interface (R892) nor with the PLC Motion I/O Interface (S525). You cannot load these options on the controller at the same time.

## LIMITATIONS

### Number of Motion Groups Supported

Prior to the V9.30P/12 (7DF3/12) and V9.40P/06 (7DF5/06) software releases, RMI only supported single group systems. RMI now supports two group motion instructions in a multiple group system.

### Hot Start is Disabled

For a controller running the Remote Motion Interface software, the controller cannot perform a Hot Start. If Hot Start is enabled, the Remote Motion Interface software disables the Hot Start when it receives the FRC Initialize command and it posts an RMIT-016 error to alert the user about the change in the system behavior.

### Original Path Resume Function is Disabled

When you hold TP program execution during motion, jog the robot away, and then press the FWD key to resume TP program execution, the TP program has two options to resume the motion:

- If the Original Path Resume function is enabled, the robot will move to the position where the HOLD button was pressed, then go to the next taught position. Essentially, the robot goes back to follow the original taught path.

- If the Original Path Resume function is disabled, the robot will not move back to the position where the robot was halted, but directly moves from the current jogged position to the next taught position.

For many RMI applications, option 2 is the preferred program resume method. For RMI applications, if you need to halt TP execution and jog away, it usually means that the robot has encountered some obstacles or other problems that require human intervention. If the robot moves back to the original path after jogging, the robot will go back to the same situation again. For this reason, RMI disables the Original Path Resume function.

### Instruction Packet Buffer

The Remote Motion Interface supports a maximum number of eight TP instructions that the remote device can send to the robot controller at one time. When the buffer is full and the robot controller

B-84184EN/03	1 INTRODUCTION

receives a new instruction packet, it returns the packet with an error code, RMIT-028. When the controller completes the first instruction, the remote device is allowed to send its next instruction.

### Minimum Motion Duration

The Remote Motion Interface does not support motion duration shorter than 12 milliseconds or 24 milliseconds, depend on robot model. If a motion instruction is too short, the robot controller slows down the motion so that the motion is at least 12 or 24 milliseconds in duration.

### Other Available Communication Options

The Remote Motion Interface is intended for control of robot motion. Some basic non-motion commands are available, but systems will typically also require a separate communications option for the exchange of data and I/O. The HMI Device option (R553) supports the use of Modbus TCP and OPC-UA, both of which are commonly used with PC-based control systems. These protocols allow for the transfer of I/O and data values between the robot and remote device.

- INTRODUCTION	B-84184EN/03

B-84184EN/03

- REMOTE MOTION APPLICATION

PROGRAM INTERFACE

# REMOTE MOTION APPLICATION PROGRAM INTERFACE

## REMOTE MOTION APPLICATION PROGRAM INTERFACE OVERVIEW

There are three types of TCP/IP packets sent from the remote device to the robot controller:

- Communication packets: These packets establish/terminate the TCP/IP communication session with the robot controller.

- Command packets: These packets perform administrative work without adding a new TPP instruction to the TP program.

- Instruction packets: These packets append TPP instructions to a running TP program.

The Remote Motion Interface uses a TCP/IP socket to communicate with the remote device where the remote device is a client and the robot controller is the server. The Remote Motion Interface uses a specific logic port and it can only talk to one remote device at one time, hence it does not allow multiple remote sources to control the same robot. The TCP/IP protocol is chosen to ensure that an instruction or command packet is always received by the intended target in the same order as it is sent. The data (payload) exchanged between the controller and the remote device is ASCII text and it is similar to a JSON string.

NOTE

Each string is terminated by \r\n so the Remote Motion Interface parser knows that a packet is a complete packet.

NOTE

The first key in the text string has to be either “Command”, “Communication” or “Instruction”. RMI will return RMIT-022 Invalid Text String if this rule is not followed.

## COMMUNICATION PACKETS

There are two connection packets that the remote device can send to the robot controller:

- FRC_Connect: Get controller permission to connect to the server. The remote device sends this packet to the controller at the beginning of a Remote Motion Interface session.

- FRC_Disconnect: Disconnect from the robot controller. This is the last packet the remote device sends to the robot controller to terminate its Remote Motion Interface (RMI) session.

2 REMOTE MOTION APPLICATION

PROGRAM INTERFACE	B-84184EN/03

### Packet Exchange

Table 2.2.1 Packet Specifications

| Remote Device Command Packet | Robot Controller Return Packet |
| --- | --- |
| {“Communication” : “FRC_Connect” } \r\n | {“Communication” : “FRC_Connect”, “ErrorID” : integerValue, “PortNumber” : shortValue1, “MajorVersion” : shortValue2, “MinorVersion” : shortValue3 } \r\n |

The remote device sends this packet to a specific robot controller port (port number 16001) to start a Remote Motion Interface session. The robot controller sends the return packet back with the integerValue = 0 to indicate that the connection is established. Otherwise, the integerValue is the robot controller’s error code.

If the robot controller allows an RMI session to start (i.e., ErrorID = 0), the PortNumber returned by the robot controller is the port number that the remote device will connect to for the rest of the RMI session. After sending the return packet, RMI automatically disconnects external device from port 16001, and no addition action is need on the external device on this port. Note that this is the only packet sent by the remote device that uses robot controller port 16001. Any other packet should use the returned shortValue1 for its TCP/IP port connection.

The robot controller also returns the major and minor version number back to the remote device. The controller changes the major version number when the controller updates the protocols defined in this section, and the controller changes its minor version number when an update is applied with no change to the protocol. In general, the protocol is backward compatible. For example, if the remote device’s interface program is implemented based on major version 1 and the major version on robot controller is 2, the remote device’s code should still work. However, if the remote device’s interface program is built on major version 2 and the controller returns that its major version is 1, the remote device interface may have issues since the controller will not provide some fields that are expected by the remote device. Therefore, it is recommended to update the robot controller to the latest software version before using the Remote Motion Interface.

Currently, the MajorVersion can have seven different values:

MajorVersion = 1

Original RMI. It does not support a controller with multiple groups and it does not support ASCII string packets defined in Chapter 5, ASCII STRING INSTRUCTION PACKETS.

MajorVersion = 2

Original RMI with support for controller with multiple groups. This version does not support ASCII string packets defined in Chapter 5, ASCII STRING INSTRUCTION PACKETS.

MajorVersion = 3

This version adds support for ASCII string packets. This new version is in V9.30P/15 (7DF3P15), V9.40P/12(7Df5P12) and later software.

Major Version = 4

B-84184EN/03

2 REMOTE MOTION APPLICATION

PROGRAM INTERFACE

This version adds support for FRC_Call, and Tool_Offset, PR[] motion modifier. The FRC_Reset command also works without running the RMI_MOVE TP program. These changes are in V9.30P/19 (7DF3P19) and V9.40P/18 (7DF5P18) and later software.

Major Version = 5

New motion modifier ALIM; AlimReg and NoBlend are added to RMI. These additional functions are available in V9.30P/24 (7DF3P24) and V9.40P/28(7DF5P28) and later software.

NOTE

You need to load option R921 (Acceleration limit) to use ALIM motion modifier.

Major Version = 6

The FRC_ReadJointAngles and FRC_ReadCartesianPosition now return the robot’s actual position. Also, the Real Time Singularity Avoidance (RTSA) option is available to allow the RMI_MOVE program to work with the Singularity Avoidance feature.

These changes are available on V9.40P/52 and V9.30P/33.

NOTE

The controller must have R792 option loaded to use RTSA in FRC_Initialize packet.

Major Version = 7

Spline motion is supported in V9.40P/54 for standard mode. In addition, the Palletizing program header is now supported in FRC_Initialize command.

NOTE

If it is possible, please update your controller to the latest version of RMI, Major Version = 7, since it includes functionalities provided by all previous versions.

### Controller Disconnect Packet

Table 2.2.2 FRC_Disconnect Packet

| Remote Device Command Packet | Robot Controller Return Packet |
| --- | --- |
| {“Communication” : “FRC_Disconnect” } \r\n | { “Communication” : “FRC_Disconnect”, “ErrorID” : integerValue } \r\n |

The remote device sends the FRC_Disconnect packet to terminate the communication session with the robot controller. The robot controller sends the return packet back with the integerValue = 0 to indicate the communication session is completed. Otherwise, the integerValue is robot controller’s error code.

2 REMOTE MOTION APPLICATION

PROGRAM INTERFACE	B-84184EN/03

### Timeout Terminate Packet

| Robot Controller Termination Packet | Remote Device return packet |
| --- | --- |
| {“Communication”:“FRC_Terminate”} \r\n | None |

When a remote device connected to the controller does not send any command/instruction packet to the controller for a long period of time (default is 60 minutes), the controller will automatically terminate the connection between the controller and the remote device. Once the connection is terminated, the remote device needs to send an FRC_Connect packet to the controller to reestablish the connection. You can change the default time by changing the system variable $rmi_cfg.$discnt_tim. If this system variable is set to 0, RMI will not check for timeout.

### System Fault Packet

| Robot Controller System Fault Packet | Remote Device Return Packet |
| --- | --- |
| {“Communication”:“FRC_SystemFault”, “SequenceID”:integerValue} \r\n | None |

The robot controller sends this packet to the remote device when the controller encounters an error and the running TP program is paused. For example, when a motion instruction causes a limit error, the controller will send out this packet to inform the remote device about this error. The integerValue is the sequence ID of the instruction that causes the issue.

## ROBOT COMMAND PACKETS

None of the packets listed in this section add an instruction to the teach pendant program. Instead, these packets ask the robot to perform functions outside of the TP program, and the effect is immediate. For example, if you send seven motion instructions to the controller and the robot is executing the third instruction, then you send a speed override command to change the program override from 100% to 50%, the override change takes effect during the execution of the third instruction and it does not wait until all seven previously sent instructions complete before changing the override.

When sending a command packet to the controller, make sure the previously sent command packet is returned. Some of the commands, such as FRC_Reset, take time to complete their operation. If you send a command packet before the controller has time to complete previous command packet , the controller will return the packet with an error code.

There are two classes of commands:

- One type only requires that the remote device has made a connection to the robot controller. That is, once the FRC_Connect packet is accepted by the controller, the controller will respond to the following commands:

  - FRC_ReadPositionRegister

  - FRC_WritePositionRegister

  - FRC_SetOverride

  - FRC_GetStatus

  - FRC_GetUFrameUTool

  - FRC_ReadUToolData

B-84184EN/03

2 REMOTE MOTION APPLICATION

PROGRAM INTERFACE

  - FRC_WriteUToolData

  - FRC_ReadUFrameData

  - FRC_WriteUFrameData

  - FRC_ReadJointAngles

  - FRC_ReadCartesianPosition

  - FRC_Reset. This command is available after the V9.30P/19 (7DF3/19) and V9.40P/18 (7DF5/18) release.

- Other commands will work depending on the RMI current status. For example, the FRC_Continue

command only works after the FRC_Initialize has been sent to the controller.

### Packet To Initialize Remote Motion

Table 2.3.1 Initialize Robot Motion Packet

| Remote Device Command Packet | Robot Controller Return Packet |
| --- | --- |
| {“Command” : “FRC_Initialize”, "GroupMask":unsignedByteValue, "RTSA" : StringValue1, "PLTZMODE": StringValue2 } \r\n | {“Command” : “FRC_Initialize”, “ErrorID” : integerValue, "GroupMask":unsignedByteValue } \r\n |
| NOTE The text with the gray background is optional. |  |

The remote device sends this packet to the controller when it wants to start adding instructions to the TP program. Before sending this command to the controller, make sure that the robot controller is in the following state:

- The teach pendant is disabled and the controller is in AUTO mode.

- The controller is ready to run, i.e., there are no servo or other errors.

- The selected TP program is not RMI_MOVE.

NOTE

Sometimes the RMI_MOVE program can be selected without the user being aware of it. If ErrorID=7015 or ErrorID=7004 is received after sending FRC_Initialize, make sure the RMI_MOVE program is not selected by doing the following: Press the SELECT button on the TP, choose another program besides RMI_MOVE from the program list, then press the ENTER button on the TP. Next, resend the FRC_Initialize command.

If you are not sure about the current state of the controller, you can send the FRC_GetStatus packet to get the current robot controller’s status.

If the initialization command is executed successfully, the return integerValue is 0. At this point, the controller has created the RMI_MOVE TP program and the program is running. Otherwise, the integerValue of the return packet is the error code from the robot controller.

2 REMOTE MOTION APPLICATION

PROGRAM INTERFACE	B-84184EN/03

If the FRC_Initialize command is executed successfully, you will have to send an FRC_Abort command to terminate the RMI program in order for another TP program to run. Otherwise, even if you manually abort the RMI_MOVE TP program through the teach pendant, the RMI still has program control and you will not be able to execute other TP programs.

It takes the robot controller some time to initialize the system to accept the TP program instructions. Please wait until you have received the return packet before sending the next packet to the controller.

The GroupMask key from a remote device is an option for a single group system. If the GroupMask key is not presented, RMI defaults the GroupMask to 1. The GroupMask is an unsigned one-byte bit- field where each bit corresponds to the group number. For example,

GroupMask = 0b00100010 (binary form)

would mean Groups 2 and 6. Since the maximum number of groups supported by a controller is eight (8), and the maximum valid value is 192 (0b11000000). However, RMI checks the value of the GroupMask against the groups that are actually used, and if the GroupMask specifies a group that does not exist in the controller’s current configuration, RMI posts the error code RMIT-040 Invalid Group Mask.

RMI can be used to control up to two (2) robot groups, so GroupMask values consisting of up to two non-zero bits are allowed. GroupMask bits of more than two non-zero bits (e.g., 0b00000111) will result in the invalid group mask error. For GroupMasks with two non-zero bits, RMI expects all the incoming motion instruction packets to have two sets of position data; one for the first group and one for the second group. If any motion instruction packet has only one group’s position data, then RMI will return an error. On the other hand, if the GroupMask is set to run one group motion and the remote device sends motion instructions with more than one group’s position data, RMI also posts an error.

For multiple group controllers, the GroupMask key is required. Otherwise, RMI will return RMIT-040

Invalid Group Mask.

NOTE

Please make sure the GroupMask is set correctly. If you set group mask to 1 and send RMI with group 2 position data, the controller will either post a run-time error due to invalid kinematics, or group 1 robot will go to the position intended for group 2 and cause unexpected motion.

The optional “RTSA” key is for real time singularity avoidance program header. If the controller has the singularity avoidance option (R792) loaded, and you want to enable singularity avoidance feature for RMI_MOVE TP program, you can set the StringValue1 to “ON”. When this option is set to “ON”, the controller will modify the path you generated to avoid robot singularity position.

The optional PLTZMODE key allows you to set the palletizing program header for the RMI_MOVE TP program. The StringValue2 has 6 options:

- ZERODN: J4 fixed angle: 0deg, Wrist axis direction: Downwards

- ZEROUP: J4 fixed angle: 0deg, Wrist axis direction: Upwards

- PSPIDN: J4 fixed angle: +180deg, Wrist axis direction: Downwards

- PSPIUP: J4 fixed angle: +180deg, Wrist axis direction: Upwards

- MSPIDN: J4 fixed angle: -180deg, Wrist axis direction: Downwards

- MSPIUP: J4 fixed angle: -180deg, Wrist axis direction: Upwards

B-84184EN/03

2 REMOTE MOTION APPLICATION

PROGRAM INTERFACE

### Packet To Abort Remote Motion TP Program

Table 2.3.2 Abort Remote Motion Packet

| Remote Device Command Packet | Robot Controller Return Packet |
| --- | --- |
| {“Command” : “FRC_Abort” } \r\n | {“Command” : “FRC_Abort”, “ErrorID” : integerValue } \r\n |

Sending the abort packet to the robot controller allows you to abort the current running RMI_MOVE TP program. After this program is aborted, the robot controller sends a return packet back to the remote device. The integerValue in the return packet is 0 if the command is successfully executed.

Otherwise, the integerValue is the controller’s error code.

NOTE

Please always end your RMI session with either an FRC_Abort or FRC_Disconnect

packet. This will ensure you can execute other TP programs after the RMI session.

### Packet To Pause Remote Motion Program

Table 2.3.3 Pause Remote Motion Packet

| Remote Device Command Packet | Robot Controller Return Packet |
| --- | --- |
| {“Command” : “FRC_Pause”} \r\n | {“Command” : “FRC_Pause”, “ErrorID” : integerValue } \r\n |

To halt the RMI_MOVE TP program execution, send this packet to pause the program. If the integerValue in the return packet is 0, the TP Program is paused successfully. Otherwise, the integerValue is the error code from the robot controller.

### Packet To Continue Remote Motion Program

Table 2.3.4 Continue Remote Motion Packet

| Remote Device Command Packet | Robot Controller Return Packet |
| --- | --- |
| {“Command” : “FRC_Continue”} \r\n | {“Command” : “FRC_Continue”, “ErrorID” : integerValue } \r\n |

To resume a halted TP program, send the continue packet to the controller to continue the TP program. If the program is continued, the returned integerValue is 0, otherwise, the integerValue contains the robot controller error code.

2 REMOTE MOTION APPLICATION

PROGRAM INTERFACE	B-84184EN/03

| “ErrorData2” : “Error String2”, |  |  |
| --- | --- | --- |
| “ErrorData3” : | “Error | String3”, |
| “ErrorData4” : | “Error | String4”, |
| “ErrorData5” : | “Error | String5”, |

Packet To Read Controller Error

Table 2.3.5 Read Controller Error Packet

| Remote Device Command Packet | Robot Controller Return Packet |
| --- | --- |
| {“Command” : “FRC_ReadError”, “Count” : byteValue1} \r\n | {“Command” : “FRC_ReadError”, “ErrorID” : integerValue, “Count” : byteValue1, “ErrorData” : “Error String” } \r\n |
| NOTE The text with the gray background is optional and is only required for reading multiple error cases. |  |

To get a controller error code, you can send the read error packet to the controller. The controller sends the most recent error text back in the return packet. The Error String has a length of 8 in a format of XXXX-NNN, where the XXXX is the sub-system error text, such as SRVO or MOTN. The NNN is the error code. Therefore, if the last error that the controller had was that the E-Stop button on the teach pendant had been pressed, the return error string will be SRVO-002.

The “Count” is an option key and its’ value has a valid range from 1 to 5. If the byteValue1 is less than 1 or greater than 5, RMI sends back a return packet with “ErrorID” = 2556949 (RMI_INV_VAL).

The FRC_ReadError will return only the most recent error text if it does not have the optional “Count” key (compatible with the original specification).

If the controller has less errors than requested, RMI changes the return packet’s byteValue1 to the number of error it can return.

B-84184EN/03

2 REMOTE MOTION APPLICATION

PROGRAM INTERFACE

### Packet To Set The Current UFrame-UTool Number

Table 2.3.6 Set Current UFrame/UTool Number Packet

| Remote Device Command Packet | Robot Controller Return Packet |
| --- | --- |
| {“Command” : “FRC_SetUFrameUTool”, “UFrameNumber” : byteValue1, “UToolNumber” : byteValue2, "Group":byteValue3 } \r\n | {“Command” : “FRC_SetUFrameUTool”, “ErrorID” : integerValue, "Group":byteValue3 } \r\n |
| NOTE The text with the gray background is optional. |  |

You can set the controller’s current user tool number and user frame number by sending this command packet to the controller, where byteValue1 is the user frame number and byteValue2 is the user tool number.

The Group key from remote device is optional. Without the Group key, RMI sets the User Frame number and User Tool number to group 1 by default.

The valid group number is from 1 to 8 since one controller can have up to 8 groups. RMI checks the input group number against the controller’s robot configuration. If the input byteValue3 is not a valid group number, RMI returns error code RMIT-039 Invalid Group Number.

NOTE

You should not send this command packet while the robot is in motion since changing the tool number and frame number may cause a frame mismatch error and stop the TP program.

2 REMOTE MOTION APPLICATION

PROGRAM INTERFACE	B-84184EN/03

### Packet To Get Controller Status

Table 2.3.7 Get Controller Status Packet

| Remote Device Command Packet | Robot Controller Return Packet |
| --- | --- |
| {“Command” : “FRC_GetStatus”} \r\n | {“Command” : “FRC_GetStatus”, “ErrorID” : integerValue, “ServoReady” : byteValue1, “TPMode” : byteValue2, “RMIMotionStatus” : byteValue3, “ProgramStatus” : byteValue4, “SingleStepMode” : byteValue5, “NumberUTool” : byteValue6, “NextSequenceID” : integerValue1, “NumberUFrame” : byteValue7, “Override” : byteValue8 } \r\n |

The return packet of the Get Current Robot Status packet has the following information:

- integerValue (ErrorID): if 0, the command is executed successfully. Otherwise, the integerValue contains the controller error code and all other values in the return packet are invalid.

- byteValue1 (ServoReady): if it is 1, the robot controller is ready for motion. Otherwise, the robot controller is in an error condition. You can try an FRC_Reset packet to clear the error.

- byteValue2 (TPMode): if it is 0, the teach pendant is disabled. If it is 1, the teach pendant is enabled. The Remote Motion interface only works when the teach pendant is disabled.

- byteValue3 (RMIMotionStatus): if 1, the Remote Motion Interface is running. If 0, the Remote Motion interface is not running and you can send a FRC_Initialize packet to restart the motion interface.

- byteValue4 (ProgramStatus):: The RMI_MOVE TP program's current status, which is defined as follows:

  - 0: RMI_MOVE program is Running.

  - 1: RMI_MOVE program is Paused.

  - 2: RMI_MOVE program is Aborted.

- byteValue5 (SingleStepMode): If 1, the robot controller is in single-step mode.

- byteValue6 (NumberUTool): Number of user tools available in the robot controller.

- byteValue7 (NumberUFrame): Number of user frames available in the robot controller.

- integerValue1 (NextSequenceID): The next valid sequence ID. This key is only valid if the system variable $RMI_CFG.$Chk_seqID = TRUE.

- byteValue8 (Override): The current override setting.

B-84184EN/03

### Packet To Read User Frame Data

2 REMOTE MOTION APPLICATION

PROGRAM INTERFACE

| Remote Device Command Packet | Robot Controller Return Packet |
| --- | --- |
| {“Command” : “FRC_ReadUFrameData”, “FrameNumber” : byteValue, "Group":byteValue2 } \r\n | {“Command” : “FRC_ReadUFrameData”, “ErrorID” : integerValue, “FrameNumber” : byteValue, “Frame” : { “X” : floatX, “Y” : floatY, “Z” : floatZ, “W” : floatW, “P” : floatP, “R” : floatR }, "Group":byteValue2 } \r\n |
| NOTE The text with the gray background is optional. |  |

You can read a user frame's data by sending a Read User Frame data packet where the byteValue is the user frame number. The return packet’s integerValue = 0 means that the position data is valid.

Otherwise, the integerValue is the controller’s error code.

The Group key from remote device is optional. Without the Group key, RMI returns the User Frame data for group 1 by default. If the input byteValue2 is not a valid group number, RMI returns error code RMIT-039 Invalid Group Number.

### Packet To Set User Frame Data

| Remote Device Command Packet | Robot Controller Return Packet |
| --- | --- |
| {“Command” : “FRC_WriteUFrameData”, “FrameNumber” : byteValue, “Frame” : {“X” : floatX, “Y” : floatY, “Z” : floatZ, “W” : floatW , “P” : floatP, “R” : FloatR }, "Group":byteValue2 } \r\n | {“Command” : “FRC_WriteUFrameData”, “ErrorID” : integerValue, "Group":byteValue2 } \r\n |
| NOTE The text with the gray background is optional. |  |

You can change a user frame's data by sending this packet. The byteValue is the user frame number. However, you should not change the user frame data when the robot is in motion since it may cause

2 REMOTE MOTION APPLICATION

PROGRAM INTERFACE	B-84184EN/03

unexpected motion. The Remote Motion Interface rejects this packet if the robot is in motion when it receives this packet.

The Group key from the remote device is optional. Without the Group key, RMI sets the user frame data for group 1 by default. If the input byteValue2 is not a valid group number, RMI returns error code RMIT-039, Invalid Group Number.

### Packet To Read User Tool Data

| Remote Device Command Packet | Robot Controller Return Packet |
| --- | --- |
| {“Command” : “FRC_ReadUToolData”, “ToolNumber” : byteValue, "Group":byteValue2 } \r\n | {“Command” : “FRC_ReadUToolData”, “ErrorID” : integerValue, “ToolNumber” : byteValue, “Frame” : { “X” : floatX, “Y” : floatY, “Z” : floatZ, “W” : floatW, “P” : floatW, “R” : floatR }, "Group":byteValue2 } \r\n |
| NOTE The text with the gray background is optional. |  |

You can read user tool data by sending this packet where the byteValue is the tool frame number. The return packet’s floatX, floatY, floatZ, floatW, floatP, floatR are the Cartesian representations of the tool frame byteValue.

The Group key from the remote device is optional. Without the Group key, RMI reads the User Tool data for group 1 by default. If the input byteValue1 is not a valid group number, RMI returns error code RMIT-039.

B-84184EN/03

### Packet To Set User Tool Data

2 REMOTE MOTION APPLICATION

PROGRAM INTERFACE

| Remote Device Command Packet | Robot Controller Return Packet |
| --- | --- |
| {“Command” : “FRC_WriteUToolData”, “ToolNumber” : byteValue, “Frame” : {“X” : floatX, “Y” : floatY, “Z” : floatZ, “W” : floatW, “P” : floatP, “R” : floatR }, "Group":byteValue2 } \r\n | {“Command” : “FRC_WriteUToolData”, “ErrorID” : integerValue, "Group":byteValue2 } \r\n |
| NOTE The text with the gray background is optional. |  |

Use this command packet to set a user tool frame, where the floatX, floatY, floatZ, floatW, floatP and floatR contain the Cartesian position data for the user tool byteValue.

The Group key from the remote device is optional. Without the Group key, RMI writes the user tool data to group 1 by default.

If the input byteValue2 is not a valid group number, RMI returns error code RMIT-039.

NOTE

You should not change the user tool frame data when the robot is in motion since it may cause unexpected motion. Please make sure that the robot is not in motion before sending this packet.

    - Packet To Read Digital Input Port

| Remote Device Command Packet | Robot Controller Return Packet |
| --- | --- |
| {“Command” : “FRC_ReadDIN”, “PortNumber” : shortValue } \r\n | {“Command” : “FRC_ReadDIN”, “ErrorID” : integerValue, “PortNumber” : shortValue, “PortValue” : byteValue } \r\n |

You can read a digital input port value using this packet, where the shortValue indicates the port number. If the return packet’s integerValue = 0, then the byteValue is the current value of DINport shortValue.

2 REMOTE MOTION APPLICATION

PROGRAM INTERFACE	B-84184EN/03

### Packet To Write Digital Output Port

| Remote Device Command Packet | Robot Controller Return Packet |
| --- | --- |
| {“Command” : “FRC_WriteDOUT”, “PortNumber” : shortValue, “PortValue” : StringValue } \r\n | {“Command” : “FRC_WriteDOUT”, “ErrorID” : integerValue } \r\n |

Use this packet to set a digital output port where the shortValue is the port number and the stringValue is either ON or OFF. In the return packet, if the integerValue = 0, then the command is successfully executed.

### Packet To Read Current Robot Cartesian Position

| Remote Device Command Packet | Robot Controller Return Packet |
| --- | --- |
| {“Command” : “FRC_ReadCartesianPosition” , "Group":byteValue } \r\n | {“Command” : “FRC_ReadCartesianPosition”, “ErrorID” : integerValue1, “TimeTag” : integerValue2, “Configuration” : { “UToolNumber” : byteValue1, “UFrameNumber” : byteValue2, “Front” : byteValue3, “Up” : byteValue4, “Left” : byteValue5, “Flip” : byteValue6, “Turn4” : byteValue7, “Turn5” : byteValue8, “Turn6” : byteValue9}, “Position” : {“X” : floatX, “Y” : float Y, “Z” : floatZ, “W”: floatW, “P” : floatP, “R” : floatR, “Ext1” : floatE1, “Ext2” : floatE2, “Ext3” : floatE3 }, "Group":byteValue } \r\n |
| NOTE The text with the gray background is optional. |  |

Use this packet to get the current robot’s Cartesian position. If the return packet’s integerValue1 = 0, the following position data are valid. Otherwise, the integerValue1 is the error code from the controller. The integervalue2 is the timestamp from the robot controller.

B-84184EN/03

2 REMOTE MOTION APPLICATION

PROGRAM INTERFACE

Note that the returned Cartesian Position are the current robot actual position, based on the encoder counts on the motors. When the robot is machine-locked, the return position will only be updated in 100 millisecond intervals. Therefore, if you are polling the robot’s actual position on every 8 millisecond intervals and the robot is machine-locked, you will get back 12 packets with the same position data. If you are testing RMI using ROBOGUIDE, the virtual robot behaves the same as machine-locked robot. Therefore, the FRC_ReadCartesianPosition will get update in every 100 milliseconds. Note: To enable machine-lock, go to Test Cycle Setup menu and set the Group Motion to DISABLE.

### Packet To Read Current Robot Joint Angles

| Remote Device Command Packet | Robot Controller Return Packet |
| --- | --- |
| {“Command” : “FRC_ReadJointAngles”, "Group":byteValue } \r\n | {“Command” : “FRC_ReadJointAngles”, “ErrorID” : integerValue1, “TimeTag” : integerValue2, “JointAngle” : {“J1” : floatJ1, “J2” : floatJ2, “J3” : floatJ3, “J4” : floatJ4, “J5” : floatJ5, “J6” : floatJ6, “J7” : floatJ7, “J8” : floatJ8, “J9” : floatJ9}, "Group": byteValue } \r\n |
| NOTE The text with the gray background is optional. |  |

Use this packet to get the current robot’s joint positions. If the return packet’s integerValue1 = 0, the following joint angle data are valid. Otherwise, the integerValue1 is the error code from the controller. Note that the returned joint angles are the current robot actual position, based on the encoder counts on the motors. When the robot is machine-locked, the return joint angles will only be updated in 100 millisecond intervals. Therefore, if you are polling the robot’s actual position on every 8 millisecond intervals and the robot is machine-locked, you will get back 12 packets with the same joint angles data. Note: To enable machine-lock, go to Test Cycle Setup menu and set the Group Motion to DISABLE.

If you are testing RMI using ROBOGUIDE, the virtual robot behaves the same as a machine-locked robot. Therefore, the FRC_ReadJointAngles will get updated every 100 milliseconds.

The Group key from the remote device is optional. Without the Group key, RMI reads the robot joint angles from group 1 by default. If the input byteValue is not a valid group number, RMI returns error code RMIT-039.

2 REMOTE MOTION APPLICATION

PROGRAM INTERFACE	B-84184EN/03

### Packet To Set Speed Override

| Remote Device Command Packet | Robot Controller Return Packet |
| --- | --- |
| {“Command” : “FRC_SetOverRide”, “Value” : byteValue } \r\n | {“Command” : “FRC_SetOverRide”, “ErrorID” : integerValue} \r\n |

Send this packet to change the program override value. The allowable range for byteValue is from 1 to

100. The return packet’s integerValue = 0 means that the command is executed successfully.

### Packet To Get Current User/Tool Frame Number

| Remote Device Command Packet | Robot Controller Return Packet |
| --- | --- |
| {“Command” : “FRC_GetUFrameUTool”, "Group":byteValue } \r\n | {“Command” : “FRC_GetUFrameUtool”, “ErrorID” : integerValue, “UFrameNumber” : byteValue1, “UToolNumber” : byteValue2, "Group":byteValue3 } \r\n |
| NOTE The text with the gray background is optional. |  |

Use this packet to get the current user frame number and user tool number of the robot controller. When the return packet’s integerValue = 0, then the byteValue1 is the current user frame number, and the byteValue2 is the current robot user tool number. If the integerValue > 0, then the integerValue is the robot’s error code and the byteValue1 and byteValue2 are invalid.

The Group key from the remote device is optional. Without the Group key, RMI reads the current user tool number and user frame number from group 1 by default. If the input byteValue is not a valid group number, RMI returns error code RMIT-039.

B-84184EN/03

2 REMOTE MOTION APPLICATION

PROGRAM INTERFACE

### Packet To Read Position Register Data

| Remote Device Command Packet | Robot Controller Return Packet |
| --- | --- |
| {“Command” : “FRC_ReadPositionRegister”, “RegisterNumber” : shortValue, "Group":byteValue } \r\n | {“Command” : “FRC_ReadPositionRegister”, “ErrorID” : integerValue, “RegisterNumber” : shortValue, “Configuration” : { “UToolNumber” : byteValue1, “UFrameNumber” : byteValue2, “Front” : byteValue3, “Up” : byteValue4, “Left” : byteValue5, “Flip”: byteValue6, “Turn4” byteValue7, “Turn5”: byteValue8, “Turn6” : byteValue9}, “Position” : {“X” : floatX, “Y” : float Y, “Z” : floatZ, “W” : floatW, “P” : floatP, “R” : floatR, “Ext1” : floatE1, “Ext2” : floatE2, “Ext3” : floatE3}, "Group": byteValue } \r\n |
| NOTE The text with the gray background is optional. |  |

Use this packet to read the position register data from the robot controller where the shortValue is the register number. If the return packet’s integerValue = 0, then the following position data is valid. Otherwise, the integerValue is the robot’s error code.

The Group key from the remote device is optional. Without the Group key, RMI returns the group 1 position from the position register by default. If the input byteValue is not a valid group number, RMI returns error code RMIT-039.

2 REMOTE MOTION APPLICATION

PROGRAM INTERFACE	B-84184EN/03

### Packet To Write Position Register Data

| Remote Device Command Packet | Robot Controller Return Packet |
| --- | --- |
| {“Command” : “FRC_WritePositionRegister”, “RegisterNumber” : shortValue, “Configuration” : { “UToolNumber” : byteValue1, “UFrameNumber” : byteValue2, “Front” : byteValue3, “Up” : byteValue4, “Left” : byteValue5, “Flip” : byteValue6, “Turn4” byteValue7, “Turn5”: byteValue8, “Turn6” : byteValue9}, “Position” : {“X” : floatX, “Y” : float Y, “Z” : floatZ, “W” : floatW, “P” : floatP, “R”: floatR, “Ext1” : floatE1, “Ext2” : floatE2, “Ext3” : floatE3}, "Group":byteValue10 } \r\n | {“Command” : “FRC_WritePositionRegister”, “ErrorID” : integerValue , “RegisterNumber” :shortValue, "Group": byteValue } \r\n |
| NOTE The text with the gray background is optional. |  |

Use this packet to write data into a position register number (shortValue). When the integerValue in the return packet = 0 it means the position register is set. Otherwise, the integerValue is the error code from the robot.

The Group key from the remote device is optional. Without the Group key, RMI writes the input position data to group 1 of the position register by default. If the input byteValue10 is not a valid group number, RMI returns error code RMIT-039.

### Packet To Reset Robot Controller

| Remote Device Command Packet | Robot Controller Return Packet |
| --- | --- |
| {“Command” : “FRC_Reset” } \r\n | {“Command” : “FRC_Reset”, “ErrorID” : integerValue} \r\n |

Use this packet to reset errors in the controller and make the robot ready to receive motion commands. When the return packet’s integerValue = 0 it indicates that the reset is successful. Otherwise, the integerValue contains the controller’s error code.

B-84184EN/03

2 REMOTE MOTION APPLICATION

PROGRAM INTERFACE

NOTE

Before V9.30P/19 (7DF3/19) and V9.40P/18 (7DF5/18), FRC_Reset only works after RMI received FRC_Initialize command. After V9.30P/19 (7DF3/19) and V9.40P/18 (7DF5/18), FRC_Reset can work without user sending FRC_Initialze command.

NOTE

It takes some time to get the system reset. Please wait until you receive the return packet before sending another command packet to the robot controller.

### Packet To Read Current Tool Center Point Speed

| Remote Device Command Packet | Robot Controller Return Packet |
| --- | --- |
| {“Command” : “FRC_ReadTCPSpeed” } \r\n | {“Command” : “FRC_ReadTCPSpeed”, “ErrorID” : integerValue1, “TimeTag” : integerValue2, “Speed” : floatValue} \r\n |

Use this packet to get the current tool center point (TCP) speed. The controller sends back a packet where the floatValue is the current TCP speed value in mm/sec.

## TEACH PENDANT PROGRAM INSTRUCTION PACKETS

When the controller receives an Initialization Packet (FRC_Initialize), it creates a TP program, RMI_MOVE, and starts the execution of this program. Please wait for the FRC_Initialize packet to be returned before sending TP instruction packets to the controller. Otherwise, these instruction packets will be returned with an error code. You can use the packets listed in this section to add a new TP instruction to the end of this running program. The controller will execute this instruction after it completes all other instructions sent previously.

The RMI_MOVE TP Program is 200 instructions in size and acts like a ring buffer. When the controller receives the 201st instruction, it puts this instruction at the beginning of the TP program and executes it when it completes the 200th instruction. Note: When the RMI_MOVE TP program is created, RMI adds two comments lines at the beginning of the TP program, so the first instruction from external device starts from line 3, that is, the third instruction in the TP program.

All the packets listed in this section add a new line of TP instruction to the RMI_MOVE TP program. You can send eight instructions to the controller at a time. However, the controller will not accept the next instruction until the first instruction is completed and a return packet is sent back to the sender, and the tenth instruction has to wait for the second instruction to complete.

Note that a motion instruction with continuous termination type (CNT1-100) will not execute until the next motion line comes in to make sure the two motions can blend together correctly. Therefore, make sure that the last motion instruction you send to the controller is with FINE termination type. Otherwise, the last motion instruction with the CNT termination type will not be executed.

2 REMOTE MOTION APPLICATION

NOTE

After version 5, RMI allows the last linear or joint motion instruction with CNT termination type to be executed if the motion has the NoBlend flag set in that instruction. The CR termination type is not affected by the NoBlend flag.

PROGRAM INTERFACE	B-84184EN/03

Each instruction packet has an ID number (SequenceID), and its value is assigned by the remote device to uniquely identify this instruction packet. This ID number should be consecutive and monotonically increasing. When this instruction is completed, the robot controller sends the return packet with the same key SequenceID and value back to the remote device so that the remote device knows which instruction has been executed.

NOTE

Start your SequenceID number from 1 after the FRC_Initialize packet. (Note that if the SequenceID should ever reach its maximum value of 231-1, it will wrap back to 1.) By default, RMI checks the SequenceID to make sure there are no missing packets. If RMI detects a non-consecutive sequence ID, RMI sends a RMIT-029 Invalid sequence ID number error ID back to the sender. At this point, RMI goes into a HOLD state. While in a HOLD state, RMI continues to execute the TP instructions that are already in the TP program but will not accept new TP instructions until RMI receives the FRC_Reset command. You can get the correct sequence ID by sending an FRC_GetStatus packet and getting “NextSequenceID” : nnnn where the nnnn is the next valid sequence ID. You can also turn off the sequence ID check by setting the system variable $RMI_CFG.$chk_seqID = FALSE.

The Remote Motion interface supports a limited set of TP instructions and they are listed here.

| Packet Name | TP Instruction Example |
| --- | --- |
| FRC_WaitDIN | WAIT DI[1] = ON |
| FRC_SetUFrame | UFRAME_NUM = 1 |
| FRC_SetUTool | UTOOL_NUM = 2 |
| FRC_WaitTime | WAIT 0.5 (sec) |
| FRC_SetPayload | PAYLOAD[2] |
| FRC_Call | Call Do_IO_Check |
| FRC_LinearMotion | L P[10] 100 mm/sec CNT100 |
| FRC_LinearRelative | L P[11] 150 mm/sec FINE INC |
| FRC_JointMotion | J P[12] 5% FINE |
| FRC_JointRelative | J P[12] 10% CNT100 INC |
| FRC_CircularMotion | C P[13] P[14] 100 mm/sec FINE |
| FRC_CircularRelative | C P[13] P[14] 100 mm/sec FINE INC |
| FRC_JointMotionJRep | J P[15] 10% CNT100 |
| FRC_JointRelativeJRep | J P[16] 5% FINE INC |

B-84184EN/03

2 REMOTE MOTION APPLICATION

PROGRAM INTERFACE

| Packet Name | TP Instruction Example |
| --- | --- |
| FRC_LinearMotionJRep | L P[10] 100 mm/sec CNT100 |
| FRC_LinearRelativeJRep | L P[11] 150 mm/sec FINE INC |
| FRC_SplineMotion | S P[19] 200 mm/sec CNT100 |
| FRC_SplineMotionJRep | S P[20] 200 mm/sec CNT100 |

Any instruction that is not in this list is not supported.

For motion instructions, the Remote Motion Interface only supports the following motion options:

- MROT

- WJnt

- Offset, PR[]

- Tool_Offset, PR[]

- COORD

- ACC

- VOFFSET, VR[]

- Local Condition Block, TA/TB/DB

- ALIM

NOTE

For V9.30P/13 (7DF3/13) and later software and V9.40P/08 (7DF5/08) and later software, RMI supports ASCII instruction strings that covers more TP instructions. Please see Chapter 5, ASCII STRING INSTRUCTION PACKETS for more details.

### Packet To Wait for DIN Instruction

| Remote Device Instruction Packet | Robot Controller Return Packet |
| --- | --- |
| {“Instruction” : “FRC_WaitDIN”, “SequenceID” : integerValue, “PortNumber” : shortValue, “PortValue” : “ON” or “OFF”} \r\n | {“Instruction” : “FRC_WaitDIN”, “ErrorID” : integerValue1, “SequenceID” : integerValue2 } \r\n |

The FRC_WaitDIN packet adds a TP instruction WAIT DI[shortValue] = ON/OFF to the TP program.

The controller sends the return packet back after the TP instruction is executed. The integerValue1 is 0 when this instruction is successfully executed. Otherwise, the integerValue1 returns an error code.

2 REMOTE MOTION APPLICATION

PROGRAM INTERFACE	B-84184EN/03

### Packet To Set User Frame Instruction

| Remote Device Instruction Packet | Robot Controller Return Packet |
| --- | --- |
| {“Instruction” : “FRC_SetUFrame”, “SequenceID” : integerValue, “FrameNumber” :byteValue } \r\n | {“Instruction” : “FRC_SetUFrame”, “ErrorID” : integerValue1, “SequenceID” : integerValue2 } \r\n |

The FRC_SetUFrame packet adds a TP instruction UFRAME_NUM = byteValue to the running TP program. The controller sends the return packet back after this instruction is executed and the RMI_Move TP program can advance to the next time. The integerValue1 is set to 0 if this instruction is successfully executed. Otherwise, the integerValue1 is the error code.

### Packet To Set User Tool Instruction

| Remote Device Instruction Packet | Robot Controller Return Packet |
| --- | --- |
| {“Instruction” : “FRC_SetUTool”, “SequenceID” : integerValue, “ToolNumber” : byteValue} \r\n | {“Instruction” : “FRC_SetUtool”, “ErrorID” : integerValue1, “SequenceID” : integerValue} \r\n |

The FRC_SetUTool packet adds a TP instruction UTOOL_NUM= byteValue to the running TP program. The controller sends the return packet back after this instruction is executed and the return integerValue1 is set to 0 if this instruction is successfully executed. Otherwise, the integerValue1 is the error code.

### Packet To Add Wait Time Instruction

| Remote Device Instruction Packet | Robot Controller Return Packet |
| --- | --- |
| {“Instruction” : “FRC_WaitTime”, “SequenceID” : integerValue, “Time” : floatValue } \r\n | {“Instruction” : “FRC_WaitTime”, “ErrorID” : integerValue1, “SequenceID” : integerValue2 } \r\n |

The FRC_WaitTime packet adds a TP instruction WAIT floatValue(sec) to the running TP program. The controller sends the return packet back after this instruction is executed. The integerValue1 is set to 0 if this instruction is successfully executed. Otherwise, the integerValue1 is the error code.

B-84184EN/03

### Packet To Set Payload Instruction

2 REMOTE MOTION APPLICATION

PROGRAM INTERFACE

| Remote Device Instruction Packet | Robot Controller Return Packet |
| --- | --- |
| {“Instruction” : “FRC_SetPayLoad”, “SequenceID” : integerValue, “ScheduleNumber” : byteValue} \r\n | {“Instruction” : “FRC_SetPayLoad”, “ErrorID” : integerValue1, “SequenceID” : integerValue} \r\n |

The FRC_PayLoad packet adds a PAYLOAD[byteValue] instruction to the running TP program. The controller sends the return packet back after this instruction is executed and the integerValue1 is set to 0 if this instruction is successfully executed. Otherwise, the integerValue1 is the error code.

### Packet To Call A Program

| Remote Device Instruction Packet | Robot Controller Return Packet |
| --- | --- |
| {“Instruction” : “FRC_Call”, “SequenceID” : integerValue, “ProgramName” : stringValue} \r\n | {“Instruction” : “FRC_Call”, “ErrorID” : integerValue1, “SequenceID” : integerValue} \r\n |

The “ProgramName” string can be up to 36 bytes in length. RMI returns an error if the String is more than 36 bytes long.

RMI sends the return packet back after the called program is executed.

2 REMOTE MOTION APPLICATION

PROGRAM INTERFACE	B-84184EN/03

    - Packet To Add Linear Motion Instruction

| Remote Device Instruction Packet | Robot Controller Return Packet |
| --- | --- |
| {“Instruction”:“FRC_LinearMotion”, “SequenceID”:integerValue, “Configuration”:{ “UToolNumber”:byteVa lue1, “UFrameNumber”:byteValue2, “Front”:byteValue3, “Up”:byteValue4, “Left”:byteValue5, “Flip”:byteValue6, “Turn4”:byteValue7, “Turn5”:byteValue8, “Turn6” : byteValue9}, “Position”:{ “X”:floatX, “Y”:floatY, “Z”:floatZ, “W”:floatW, “P”:floatP, “R”:floatR, “Ext1”:floatE1, “Ext2”:floatE2, “Ext3”:floatE3}, “SpeedType”:stringValue1, “Speed”:shortValue1, “TermType”:stringValue2, “TermValue”:byteValue10, | {“Instruction”:“FRC_LinearMotion”, “ErrorID”:integerValue1, “SequenceID”:integerValue2 } \r\n |
| The following items are optional. “ACC”:byteValue11, “OffsetPRNumber”:shortValue2, “VisionPRNumber”:shortValue3, “WristJoint”:“ON”, “MROT”:“ON”, “LCBType”:stringValue3, “LCBValue”:shortValue4, “PortType”:byteValue12, “portNumber”:shortValue5, “portValue”:stringValue4, “ToolOffsetPRNumber”:shortValue6, “ALIM”:integerValue1, “ALIMREG”:shortValue7, “NoBlend”:“ON”} \r\n |  |

B-84184EN/03

2 REMOTE MOTION APPLICATION

PROGRAM INTERFACE

The FRC_LinearMotion packet adds a linear motion instruction to the running TP program. The basic motion command has the following items that you can change:

- SpeedType: the stringValue1 can be

  - mmSec : mm/sec

  - InchMin : 0.1 inch/min

  - Time : 0.1 seconds

  - mSec : milliseconds

- Speed : shortValue1 is the speed value.

- TermType : stringValue2 can be either FINE , CNT or CR. If the TermType is FINE, the

byteValue10 is ignored.

- TermValue : byteValue10 is the continuous termination type; value from 1-100.

NOTE

The CR TermType is only valid when the Advanced Constant Path option is loaded. If the robot does not have this option, the controller returns an error. CR is similar to the CNT termination type. It requires the next motion instruction to plan its motion. Therefore, please do not use the CR termination type for the last motion you send to the controller, since this motion will not be executed.

For example, assuming the stringValue1 = mmsec, shortValue1 = 150 with CNT termination type and byteValue10 = 100; then the following instruction is added to the TP program:

L P[10] 150mm/sec CNT100

As previously noted, the optional items above can be added to the motion instruction. The following specifications apply to these options:

- ACC : byteValue11. You can add an ACC option to the motion instruction, where the valid byteValue11 is from 20 to 100. Assuming byteValue11 is 80, then the added instruction is L P[10] 150 mm/sec CNT100 ACC80.

- OffsetPRNumber : shortValue2. You can also add a position register offset to the motion instruction. The shortValue2 is the register number. Assuming the shortValue2 = 2, then the instruction becomes L P[10] 100 mm/sec CNT100 Offset, PR[2].

- ToolOffsetPRNumber : shortValue6. You can also add a tool position register offset to the motion instruction. The shortValue6 is the register number. Assuming the shortValue2 = 2, then the instruction becomes L P[10] 100 mm/sec CNT100 Tool_Offset, PR[2].

- VisionPRNumber : shortValue3 adds a vision offset to the linear motion. The shortValue3 is the vision offset position register number. Assuming the shortValue3 = 5 then the linear motion instruction becomes L P[10] 100 mm/sec CNT100 VOFFSET, VR[5].

- WristJoint : ON adds a Wrist Joint motion option to the linear motion instruction.

NOTE

This option only applies to Linear and Circular motion and it is ignored when used in a joint motion instruction.

With this option, the linear motion instruction becomes L P[10] 100 mm/sec CNT100 Wjnt .

- MROT : ON adds a MROT motion option to the linear motion instruction with Wrist Joint option. With this option, the linear motion instruction becomes:

L P[10] 100 mm/sec CNT100 Wjnt MROT

2 REMOTE MOTION APPLICATION

NOTE

The MROT is available only when the R640 option is loaded on the controller. Otherwise, the controller will return an error for this instruction.

For linear motion and circular motion, MROT only works with Wrist Joint option. Without Wrist joint option, MROT cause an error for both linear or circular motion.

PROGRAM INTERFACE	B-84184EN/03

- LCBType : stringValue3 = TB, DB, or TA: There are three LCB (Local Condition Block) types:

  - TB: Time before the robot reaches the destination point. The unit for TB is seconds.

  - TA: Time after the robot reaches the destination point. The unit for TA is seconds.

  - DB: Distance before the robot reaches the destination point. The unit for DB is mm.

The LCB is used to trigger an output port when the robot reaches a certain position with respect to the destination position.

  - LCBValue : shortValue4. This shortValue4 indicates the TB/TA/DB value. For DB, the unit is 0.01 mm. For TA and TB , the unit is ms.

  - PortType : byteValue12. The byteValue12 can be either

    - 1: DOUT

    - 2: ROUT

NOTE

GO and AO are not supported

  - portNumber : shortValue5: The shortValue5 is the DOUT/ROUT port number.

  - portValue : stringValue4. The string can either be ON or OFF.

For example, given the LCBType : TA and LCBValue : 100, PortType : 1, portNumber : 3 and

portValue : ON, the motion instruction becomes:

L P[10] 150mm/sec CNT10 TA 0.10 sec DO[3] = ON

After the controller executes this linear motion instruction, the controller sends the return packet back. If the integerValue1 = 0 it means that the motion instruction has been successfully executed. If integerValue1 > 0 it means that there was an error while executing this motion and the integerValue1 is the error code.

NOTE

The MROT, Offset PR, and Offset VR options change the robot’s trajectory. Therefore, if these settings are invalid, the controller will not insert these motion instructions in the TP program and will return the motion instruction immediately. At this point, the robot will not accept any new TP instructions until the controller receives an FRC_Reset command.

- ALIM : integerValue1 adds an Acceleration Limit motion modifier to the motion instruction. The unit is in mm/sec^2 and the maximum allowed value is 20000. This option is valid only when the controller has R921 “Acceleration Limit” option loaded.

- ALIMREG : shortValue7 adds an Acceleration Limit with Register motion modifier to the motion instruction. If the shortValue7 is less than 1 and bigger than existing register number, the program execution will post an error and stop program execution. If the register is not initialized, program execution stops and an error is posted.

- NoBlend : ON allows a motion with CNT termination type to be executed without waiting for next motion. By default. NoBlend is always OFF.

B-84184EN/03

2 REMOTE MOTION APPLICATION

PROGRAM INTERFACE

NOTE

This is a special motion modifier for RMI only, It does not add any text to the motion instruction.

RMI provides abbreviations for some of the key strings. These abbreviations are listed below:

- “SequenceID” (“SID”)

- “Turn4” (“T4”)

- “Turn5” (“T5”)

- “Turn6” (“T6”)

- “UToolNumber” (“UTNum”)

- “UFrameNumber” (“UFNum”)

NOTE

If your controller does not have extended axes, you can ignore the “Ext1”, “Ext2” and “Ext3” keys in the Position.

2 REMOTE MOTION APPLICATION

PROGRAM INTERFACE	B-84184EN/03

#### Packet to add two groups linear motion instruction

| Remote Device Instruction Packet | Robot Controller Return Packet |
| --- | --- |
| {“Instruction” : “FRC_LinearMotion”, | {“Instruction” : “FRC_LinearMotion”, |
| “SequenceID” : integerValue, | “ErrorID” : integerValue1, |
| “G1” : { | “SequenceID” : integerValue2 } \r\n |
| “Configuration” : { |  |
| “UToolNumber” : byteValue1, |  |
| “UFrameNumber” : byteValue2, |  |
| “Front” : byteValue3, “Up” : byteValue4, |  |
| “Left” : byteValue5, “Flip” : byteValue6, |  |
| “T4”: byteValue7, |  |
| “T5” : byteValue8, |  |
| “T6” : byteValue9}, |  |
| “Position” : {“X” : floatX, “Y” : floatY, |  |
| “Z” : floatZ,“W” : floatW, “P” : floatP, |  |
| “R” : floatR1,“Ext1” : floatE1, “Ext2” : floatE2, “Ext3” : floatE3}}, |  |
| “G2” : { |  |
| “Configuration” : { |  |
| “UToolNumber” : byteValue10, |  |
| “UFrameNumber” : byteValue11, |  |
| “Front” : byteValue12, “Up” : byteValue13, |  |
| “Left” : byteValue14, “Flip” : byteValue15, |  |
| “T4”: byteValue16, |  |
| “T5” : byteValue17, |  |
| “T6” : byteValue18}, |  |
| “Position” : {“X” : floatX2, “Y” : floatY2, |  |
| “Z” : floatZ2,“W” : floatW2, “P” : floatP2, |  |
| “R” : floatR2,“Ext1” : floatE4, “Ext2” : floatE5, |  |
| “Ext3” : floatE6}}, |  |
| “SpeedType” : stringValue1, |  |
| “Speed” : shortValue1, |  |
| “TermType” : stringValue2, |  |
| “TermValue” : byteValue19, |  |

B-84184EN/03

2 REMOTE MOTION APPLICATION

PROGRAM INTERFACE

| Remote Device Instruction Packet | Robot Controller Return Packet |
| --- | --- |
| The following items are optional: “ACC” : byteValue20, “OffsetPRNumber” : shortValue2, “VisionPRNumber” : shortValue3, “WristJoint” : stringValue3 : “ON” or “OFF”, “MROT” : stringValue4 : “ON” or “OFF”, “Coord” : stringValue5 : “ON” or “OFF”, “LCBType” : stringValue6, “LCBValue” : shortValue4, “PortType” : byteValue21, “PortNumber” : shortValue5, “PortValue” : stringValue7, “ToolOffsetPRNumber” : shortValue6, “NoBlend” : “ON” } \r\n |  |

NOTE

G1 indicates group 1 position and G2 is for group 2. For controllers with more than two groups, G3, G4, G5 and G6 are valid as long as the group mask is set accordingly. For example, if the FRC_Initialize packet has GroupMask = 5, RMI expects to see both G1 and G3 in all RMI motion instructions sent to the controller. All other combinations of Gx are invalid.

The first group position should have the same representation as required by the packet. For example, G1 has to have Cartesian position representation when it is the first position data in the FRC_LinearMotion packet. The second position can be either Cartesian representation or joint representation. RMI does not check the second position’s representation type. If you have position data such that G1 has joint angles and G2 has Cartesian position, you can still use FRC_LinearMotion with G2 be the first position data in the packet.

The Coord option is valid only if the RMI_MOVE TP program has 2 groups. Otherwise, an error is returned to the remote device. RMI does not check the coordinated motion setup when it receives the COORD option key, it is user’s responsibility to set up the coordinate motion pair before running the RMI. Otherwise, you will get run time error posted by the motion system.

If the second group is a positioner, the FRC_LinearMotion Instruction should be as follows:

2 REMOTE MOTION APPLICATION

PROGRAM INTERFACE	B-84184EN/03

| Remote Device Instruction Packet | Robot Controller Return Packet |
| --- | --- |
| {“Instruction” : “FRC_LinearMotion”, | {“Instruction” : “FRC_LinearMotion”, |
| “SequenceID” : integerValue, | “ErrorID” : integerValue1, |
| “G1” : { | “SequenceID” : integerValue2} \r\n |
| “Configuration” : { |  |
| “UToolNumber” : byteValue1, |  |
| “UFrameNumber” : byteValue2, |  |
| “Front” : byteValue3, |  |
| “Up” : byteValue4, |  |
| “Left” : byteValue5, |  |
| “Flip” : byteValue6, “T4”: byteValue7, |  |
| “T5” : byteValue8, “T6” : byteValue9}, |  |
| “Position” : { “X” : floatX, “Y” : floatY, “Z” : floatZ, “W” : floatW, “P” : floatP, “R” : floatR, |  |
| “Ext1” : floatE1, “Ext2” : floatE2, “Ext3” : floatE3} |  |
| }, |  |
| “G2” : { |  |
| “JointAngle” : { “J1” : floatJ1, “J2” : floatJ2, |  |
| “J3” : floatJ3, “J4” : floatJ4, “J5” : floatJ5, |  |
| “J6” : floatJ6, “J7” : floatJ7, “J8” : floatJ8, |  |
| “J9” : floatJ9 } |  |
| }, |  |
| “SpeedType” : stringValue1, |  |
| “Speed” : shortValue1, |  |
| “TermType” : stringValue2, |  |
| “TermValue” : byteValue10, |  |

B-84184EN/03

2 REMOTE MOTION APPLICATION

PROGRAM INTERFACE

| Remote Device Instruction Packet | Robot Controller Return Packet |
| --- | --- |
| The following items are optional: “ACC” : byteValue11, “OffsetPRNumber” : shortValue2, “VisionPRNumber” : shortValue3, “WristJoint” : stringValue3: “ON” or “OFF”, “MROT” : stringValue4: “ON” or “OFF”, “COORD” : stringValue5: “ON” or “OFF”, “LCBType” : stringValue6, “LCBValue” : shortValue4, “PortType” : byteValue12, “portNumber” : shortValue5, “portValue” : stringValue7, “ToolOffsetPRNumber” : shortValue6, “NoBlend” : “ON” } \r\n |  |

NOTE

JointAngle does not have to have all nine axes defined. If the positioner has only two axes, the group 2 can be simplified as “G2” : { “JointAngle” : { “J1” : floatJ1, “J2” : floatJ2 } }, However, if the positioner has two axes and the remote device only provides one joint angle, RMI assumes J2 is at 0.

2 REMOTE MOTION APPLICATION

PROGRAM INTERFACE	B-84184EN/03

    - Packet To Add Linear Incremental Motion Instruction

| Remote Device Instruction Packet | Robot Controller Return Packet |
| --- | --- |
| {“Instruction” : “FRC_LinearRelative”, “SequenceID” : integerValue, “Configuration” : { “UToolNumber” : byteValue1, “UFrameNumber” : byteValue2, “Front” : byteValue3, “Up” : byteValue4, “Left” : byteValue5, “Flip” : byteValue6, “Turn4” : byteValue7, “Turn5” : byteValue8, “Turn6” : byteValue9 }, “Position” : { “X” : floatX, “Y” : floatY, “Z” : floatZ, “W” : floatW, “P” : floatP, “R” : floatR, “Ext1” : floatE1, “Ext2” : floatE2, “Ext3” : floatE3}, “SpeedType” : stringValue1, “Speed” : shortValue1, “TermType” : stringValue2, “TermValue” : byteValue10, | {“Instruction” : “FRC_LinearRelative”, “ErrorID” : integerValue1, “SequenceID” : integerValue2 } \r\n |
| The following are optional: “ACC” : byteValue11, “OffsetPRNumber” : shortValue2, “VisionPRNumber” : shortValue3, “WristJoint”: “ON”, “MROT” : “ON”, “LCBType” : stringValue3, “LCBValue” : shortValue4, “PortType” : byteValue12, “portNumber” : shortValue5, “portValue” : stringValue4, “ToolOffsetPRNumber” : shortValue6, “ALIM”:integerValue1, “ALIMREG”:shortValue7, “NoBlend”:“ON”} \r\n |  |

B-84184EN/03

2 REMOTE MOTION APPLICATION

PROGRAM INTERFACE

The FRC_LinearRelative packet is the same as the FRC_LinearMotion, except the position data is a relative position. For a simple FRC_LinearRelative packet, the controller creates a TP line as:

L P[10] 150mm/sec CNT100 INC

After the controller executes this incremental linear motion instruction, the controller sends the return packet back. The return value integerValue1 = 0 means that the motion instruction has been successfully executed. If integerValue1 > 0, there was an error in executing the motion and the integerValue1 is the error code.

NOTE

If your controller does not have extended axes, you can ignore the “Ext1”, “Ext2” and “Ext3” keys in the Position.

#### Packet to add two groups linear incremental motion instruction

The two groups FRC_LinearRelative instruction shares the same protocol as the two groups FRC_LinearMotion instruction defined in Section 2.4.7.1, Packet to add two groups linear motion instruction. Please use the protocol defined in Section 2.4.7.1, Packet to add two groups linear motion instruction, change the value of the Instruction key from FRC_LinearMotion to FRC_LinearRelative, replace the absolute position data with incremental position and you have a two groups linear incremental motion packet for RMI.

2 REMOTE MOTION APPLICATION

PROGRAM INTERFACE	B-84184EN/03

    - Packet To Add Joint Motion Instruction

| Remote Device Instruction Packet | Robot Controller Return Packet |
| --- | --- |
| {“Instruction” : “FRC_JointMotion”, “SequenceID” : integerValue, “Configuration” : { “UToolNumber” : byteValue1, “UFrameNumber” : byteValue2, “Front” : byteValue3, “Up” : byteValue4, “Left” : byteValue5, “Flip” : byteValue6, “Turn4” : byteValue7, “Turn5” : byteValue8, “Turn6” : byteValue9 }, “Position” : { “X” : floatX, “Y” : floatY, “Z” : floatZ, “W” : floatW, “P” : floatP, “R” : floatR, “Ext1” : floatE1, “Ext2” : floatE2, “Ext3” : floatE3}, “SpeedType” : stringValue1, “Speed” : shortValue1, “TermType” : stringValue2, “TermValue” : byteValue10, | {“Instruction” : “FRC_JointMotion”, “ErrorID” : integerValue1, “SequenceID” : integerValue2 } \r\n |
| The following are optional: “ACC” : byteValue11, “OffsetPRNumber” : shortValue2, “VisionPRNumber” : shortValue3, “MROT” : “ON”, “LCBType” : stringValue3, “LCBValue” : shortValue4, “PortType” : byteValue12, “portNumber” : shortValue5, “portValue” : stringValue4, “ToolOffsetPRNumber” : shortValue6, “NoBlend” : “ON” } \r\n |  |

The FRC_JointMotion packet is the same as the FRC_LinearMotion, except that the motion type is joint instead of linear.

In addition, the joint motion only supports the following two speed types:

B-84184EN/03

2 REMOTE MOTION APPLICATION

PROGRAM INTERFACE

- Percent : %

- Time : 0.1 sec

- mSec : milliseconds

For a simple FRC_JointMotion packet, the controller creates a TP line such as:

J P[10] 10% CNT100

After the controller executes this joint motion instruction, the controller sends the return packet back. When the return value integerValue1 = 0 it means that the motion instruction has been successfully executed. If integerValue1 > 0, there is an error in executing the motion and the integerValue1 is the error code.

NOTE

If your controller does not have extended axes, you can ignore the “Ext1”, “Ext2” and “Ext3” keys in the Position.

2 REMOTE MOTION APPLICATION

PROGRAM INTERFACE	B-84184EN/03

#### Packet to add two group joint motion instruction

| Remote Device Instruction Packet | Robot Controller Return Packet |
| --- | --- |
| {“Instruction” : “FRC_JointMotion”, “SequenceID” : integerValue, “G1” : { “Configuration” : { “UToolNumber” : byteValue1, “UFrameNumber” : byteValue2, “Front” : byteValue3, “Up” : byteValue4, “Left” : byteValue5, “Flip” : byteValue6, “T4” : byteValue7, “T5” : byteValue8, “T6” : byteValue9 }, “Position” : { “X” : floatX1, “Y” : floatY1, “Z” : floatZ1, “W” : floatW1, “P” : floatP1, “R” : floatR1, “Ext1” : floatE1, “Ext2” : floatE2, “Ext3” : floatE3}}, “G2” : { “Configuration” : { “UToolNumber” : byteValue10, “UFrameNumber” : byteValue11, “Front” : byteValue12, “Up” : byteValue13, “Left” : byteValue14, “Flip” : byteValue15, “T4” : byteValue16, “T5” : byteValue17, “T6” : byteValue18 }, “Position” : { “X” : floatX2, “Y” : floatY2, “Z” : floatZ2, “W” : floatW2, “P” : floatP2, “R” : floatR2, “Ext1” : floatE4, “Ext2” : floatE5, “Ext3” : floatE6} }, “SpeedType” : stringValue1, “Speed” : shortValue1, “TermType” : stringValue2, “TermValue” : byteValue19, | {“Instruction” : “FRC_JointMotion”, “ErrorID” : integerValue1, “SequenceID” : integerValue2 } \r\n |

B-84184EN/03

2 REMOTE MOTION APPLICATION

PROGRAM INTERFACE

| Remote Device Instruction Packet | Robot Controller Return Packet |
| --- | --- |
| The following are optional: “ACC” : byteValue20, “OffsetPRNumber” : shortValue2, “VisionPRNumber” : shortValue3, “MROT” : stringValue3: “ON” or “OFF”, “LCBType” : stringValue4, “LCBValue” : shortValue4, “PortType” : byteValue21, “portNumber” : shortValue5, “portValue” : stringValue5, “ToolOffsetPRNumber” : shortValue6, “NoBlend” : “ON” } \r\n |  |

NOTE

G1 indicates group 1 position and G2 is for group 2. For controller with more than two groups, G3, G4 are valid as long as the group mask is set accordingly by the FRC_Initialize command.

NOTE

Joint motion does not support the COORD motion option. If you specify COORD in a joint motion instruction, RMI will return an error code.

2 REMOTE MOTION APPLICATION

PROGRAM INTERFACE	B-84184EN/03

    - Packet To Add Joint Incremental Motion Instruction

| Remote Device Instruction Packet | Robot Controller Return Packet |
| --- | --- |
| {“Instruction” : “FRC_JointRelative”, “SequenceID” : integerValue, “Configuration”: { “UToolNumber” : byteValue1, “UFrameNumber” : byteValue2, “Front” : byteValue3, “Up” : byteValue4, “Left” : byteValue5, “Flip” : byteValue6, “Turn4” : byteValue7, “Turn5” : byteValue8, “Turn6” : byteValue9 }, “Position” : { “X” : floatX, “Y” : floatY, “Z” : floatZ, “W” : floatW, “P” : floatP, “R” : floatR, “Ext1” : floatE1, “Ext2” : floatE2, “Ext3” : floatE3}, “SpeedType” : stringValue1, “Speed” : shortValue1, “TermType” : stringValue2, “TermValue” : byteValue10, | {“Instruction” : “FRC_JointRelative”, “ErrorID” : integerValue1, “SequenceID” : integerValue2 } \r\n |
| The following are optional: “ACC” : byteValue11, “OffsetPRNumber” : shortValue2, “VisionPRNumber” : shortValue3, “MROT” : “ON”, “LCBType” : stringValue3, “LCBValue” : shortValue4, “PortType” : byteValue12, “portNumber” : shortValue5, “portValue” : stringValue4, “ToolOffsetPRNumber” : shortValue6, “NoBlend” : “ON” } \r\n |  |

The FRC_JointRelative packet is the same as FRC_JointMotion, except the position is a relative position. For a simple FRC_JointRelative packet, the controller creates a TP line as:

J P[10] 10% CNT100 INC

B-84184EN/03

2 REMOTE MOTION APPLICATION

PROGRAM INTERFACE

After the controller executes this incremental joint motion instruction, the controller sends the return packet back. The return value integerValue1 = 0 means that the motion instruction was successfully executed. If integerValue1 > 0, there is an error in executing the motion and the integerValue1 is the error code.

NOTE

If your controller does not have extended axes, you can ignore the “Ext1”, “Ext2” and “Ext3” keys in the Position.

#### Packet to add two groups joint incremental motion instruction

The two groups the FRC_JointRelative instruction shares the same protocol as the two groups FRC_JointMotion instruction defined in Section 2.4.9.1, Packet to add two group joint motion instruction. Please use the same protocol defined in Section 2.4.9.1, Packet to add two group joint motion instruction, change the value of the Instruction key from FRC_JointrMotion to FRC_JointRelative, replace the absolute joint angles with incremental joint angles and you have the two groups joint incremental motion packet for RMI.

2 REMOTE MOTION APPLICATION

PROGRAM INTERFACE	B-84184EN/03

    - Packet To Add Circular Motion Instruction

| Remote Device Instruction Packet | Robot Controller Return Packet |
| --- | --- |
| {“Instruction” : “FRC_CircularMotion”, “SequenceID” : integerValue, “Configuration” : { “UToolNumber” : byteValue1, “UFrameNumber” : byteValue2, “Front” : byteValue3, “Up” : byteValue4, “Left” : byteValue5, “Flip” : byteValue6, “Turn4” : byteValue7, “Turn5” : byteValue8, “Turn6” : byteValue9}, “Position” : { “X” : floatX, “Y” : floatY, “Z” : floatZ, “W” : floatW, “P” : floatP, “R” : floatR , “Ext1” : floatE1, “Ext2” : floatE2, “Ext3” : floatE3}, “ViaConfiguration” : { “UtoolNumber” : byteValue10, “UFrameNumber” : byteValue11, “Front” : byteValue12, “Up” : byteValue13, “Left” : byteValue14, “Flip” : byteValue15, “Turn4” : byteValue16, “Turn5” : byteValue17, “Turn6” : byteValue18 }, “ViaPosition” : { “X” : floatX, “Y” : floatY, “Z” : floatZ, “W” : floatW, “P” : floatP, “R” : floatR, “Ext1” : floatE4, “Ext2” : floatE5, “Ext3” : floatE6}, “SpeedType” : stringValue1, “Speed” : shortValue1, “TermType” : stringValue2, “TermValue” : byteValue19, | {“Instruction” : “FRC_CircularMotion”, “ErrorID” : integerValue1, “SequenceID” : integerValue2 } \r\n |

B-84184EN/03

2 REMOTE MOTION APPLICATION

PROGRAM INTERFACE

| Remote Device Instruction Packet | Robot Controller Return Packet |
| --- | --- |
| The following are optional: “ACC” : byteValue20, “OffsetPRNumber” : shortValue2, “VisionPRNumber” : shortValue3, “WristJoint” : “ON”, “MROT” : “ON”, “LCBType” : stringValue3, “LCBValue” : shortValue4, “PortType” : byteValue21, “portNumber” : shortValue5, “portValue” : stringValue4, “ToolOffsetPRNumber” : shortValue6, “NoBlend” : “ON” } \r\n |  |

The FRC_CircularMotion packet has two sets of position data: Destination position data and via position data, as shown in the packet. This packet adds a circular motion instruction to the running TP program:

C P[10]

P[11] 15mm/sec CNT100

The circular motion supports the same optional keys as the linear motion. Please refer to the linear motion instruction for a detailed description.

After the controller executes this circular motion instruction, the controller sends the return packet back. The return value integerValue1 = 0 means that the motion instruction was successfully executed. If integerValue1 > 0, there is an error in executing the motion and the integerValue1 is the error code.

NOTE

If your controller does not have extended axes, you can ignore the “Ext1”, “Ext2” and “Ext3” keys in the Position.

2 REMOTE MOTION APPLICATION

PROGRAM INTERFACE	B-84184EN/03

#### Packet to add two groups circular motion instruction

| Remote Device Instruction Packet | Robot Controller Return Packet |
| --- | --- |
| {“Instruction” : “FRC_CircularMotion”, “SequenceID” : integerValue, “G1” : { “Configuration” : { “UToolNumber” : byteValue1, “UFrameNumber” : byteValue2, “Front” : byteValue3, “Up” : byteValue4, “Left” : byteValue5, “Flip” : byteValue6, “T4” : byteValue7, “T5” : byteValue8, “Turn6” : byteValue9}, “Position” : { “X” : floatX1, “Y” : floatY1, “Z” : floatZ1, “W” : floatW1, “P” : floatP1, “R” : floatR1 , “Ext1” : floatE1, “Ext2” : floatE2, “Ext3” : floatE3}, “ViaConfiguration” : { “UToolNumber” : byteValue10, “UFrameNumber” : byteValue11, “Front” : byteValue12, “Up” : byteValue13, “Left” : byteValue14, “Flip” : byteValue15, “T4” : byteValue16, “T5” : byteValue17, “T6” : byteValue18 }, “ViaPosition” : { “X” : floatX2, “Y” : floatY2, “Z” : floatZ2, “W” : floatW2, “P” : floatP2, “R” : floatR2, “Ext1” : floatE4, “Ext2” : floatE5, “Ext3” : floatE6} }, “G2” : { “Configuration” : { “UToolNumber” : byteValue19, “UFrameNumber” : byteValue20, “Front” : byteValue21, “Up” : byteValue22, “Left” : byteValue23, “Flip” : byteValue24, “T4” : byteValue25, “T5” : byteValue26, “Turn6” : byteValue27}, “Position” : { “X” : floatX3, “Y” : floatY3, “Z” : floatZ3, “W” : floatW3, “P” : floatP3, “R” : floatR3 , “Ext1” : floatE7, “Ext2” : floatE8, “Ext3” : floatE9}, “ViaConfiguration” : { | {“Instruction” : “FRC_CircularMotion”, “ErrorID” : integerValue1, “SequenceID” : integerValue2 } \r\n |

B-84184EN/03

2 REMOTE MOTION APPLICATION

PROGRAM INTERFACE

| Remote Device Instruction Packet | Robot Controller Return Packet |
| --- | --- |
| “UToolNumber” : byteValue28, “UFrameNumber” : byteValue29, “Front” : byteValue30, “Up” : byteValue31, “Left” : byteValue32, “Flip” : byteValue33, “T4” : byteValue34, “T5” : byteValue35, “T6” : byteValue36 }, “ViaPosition” : { “X” : floatX4, “Y” : floatY4, “Z” : floatZ4, “W” : floatW4, “P” : floatP4, “R” : floatR4, “Ext1” : floatE10, “Ext2” : floatE11, “Ext3” : floatE12} }, “SpeedType” : stringValue1, “Speed” : shortValue1, “TermType” : stringValue2, “TermValue” : byteValue37, |  |
| The following are optional: “ACC” : byteValue38, “OffsetPRNumber” : shortValue2, “VisionPRNumber” : shortValue3, “WristJoint” : stringValue3: “ON” or “OFF”, “MROT” : stringValue4: “ON” or “OFF”, “COORD”: stringValue5: “ON” or “OFF”, “LCBType” : stringValue6, “LCBValue” : shortValue4, “PortType” : byteValue39, “portNumber” : shortValue5, “portValue” : stringValue7, “ToolOffsetPRNumber” : shortValue6, “NoBlend” : “ON” } \r\n |  |

2 REMOTE MOTION APPLICATION

PROGRAM INTERFACE	B-84184EN/03

    - Packet To Add Circular Incremental Motion Instruction

| Remote Device Instruction Packet | Robot Controller Return Packet |
| --- | --- |
| {“Instruction” : “FRC_CircularRelative”, “SequenceID” : integerValue, “Configuration” : { “UToolNumber” : byteValue1, “UFrameNumber” : byteValue2, “Front” : byteValue3, “Up” : byteValue4, “Left” : byteValue5, “Flip” : byteValue6, “Turn4” : byteValue7, “Turn5” : byteValue8, “Turn6” : byteValue9 }, “Position”: { “X” : floatX, “Y” : floatY, “Z” : floatZ, “W” : floatW, “P” : floatP, “R” : floatR, “Ext1” : floatE1, “Ext2” : floatE2, “Ext3” : floatE3}, “ViaConfiguration” : { “UtoolNumber” : byteValue10, “UFrameNumber” : byteValue11, “Front” : byteValue12, “Up” : byteValue13, “Left” : byteValue14, “Flip” : byteValue15, “Turn4” : byteValue16, “Turn5” : byteValue17, “Turn6” : byteValue18}, “ViaPosition” : { “X” : floatX, “Y” : floatY, “Z” : floatZ, “W” : floatW, “P”: floatP, “R”: floatR, “Ext1” : floatE4, “Ext2” : floatE5, “Ext3” : floatE6}, “SpeedType”: stringValue1, “Speed” : shortValue1, “TermType” : stringValue2, “TermValue” : byteValue19, | {“Instruction” : “FRC_CircularRelative”, “ErrorID” : integerValue1, “SequenceID” : integerValue2 } \r\n |

B-84184EN/03

2 REMOTE MOTION APPLICATION

PROGRAM INTERFACE

| Remote Device Instruction Packet | Robot Controller Return Packet |
| --- | --- |
| The following items are optional: “ACC” : byteValue20, “OffsetPRNumber” : shortValue2, “VisionPRNumber” : shortValue3, “WristJoint” : “ON”, “MROT” : “ON”, “LCBType” : stringValue3, “LCBValue” : shortValue4, “PortType” : byteValue21, “portNumber” : shortValue5, “portValue” : stringValue4, “ToolOffsetPRNumber” : shortValue6, “NoBlend” : “ON” } \r\n |  |

The FRC_CircularRelative packet is the same as the FRC_CircularMotion, except the destination and via position are relative. This packet adds the following line to the running TP program:

C P[10]

P[11] 15mm/sec CNT100 INC

After the controller executes this incremental circular motion instruction, the controller sends the return packet back. If the return value integerValue1 = 0 it means that the motion instruction was successfully executed. If integerValue1 > 0, there was an error in executing the motion and the integerValue1 is the error code.

NOTE

If your controller does not have extended axes, you can ignore the “Ext1”, “Ext2” and “Ext3” keys in the Position.

#### Packet to add two groups circular incremental motion instruction

The two groups FRC_CircularRelative instruction shares the same protocol as the two groups FRC_CircularMotion instruction defined in Section 2.4.11.1, Packet to add two groups circular motion instruction. Please use the same protocol defined in Section 2.4.11.1, Packet to add two groups circular motion instruction, change the value of the Instruction key from FRC_CircularMotion to FRC_CircularRelative , replace the absolute Cartesian position with incremental Cartesian position and you have the two groups circular incremental motion packet for RMI.

2 REMOTE MOTION APPLICATION

PROGRAM INTERFACE	B-84184EN/03

    - Packet To Add Joint Motion With Joint Representation

| Remote Device Instruction Packet | Robot Controller Return Packet |
| --- | --- |
| {“Instruction” : “FRC_JointMotionJRep”, “SequenceID” : integerValue, “JointAngle” : { “J1” : floatJ1, “J2” : floatJ2, “J3” : floatJ3, “J4” : floatJ4, “J5” : floatJ5, “J6” : floatJ6, “J7” : floatJ7, “J8” : floatJ8, “J9” : floatJ9}, “SpeedType” : stringValue1, “Speed” : shortValue1, “TermType” : stringValue2, “TermValue” : byteValue1, | {“Instruction” : “FRC_JointMotionJRep”, “ErrorID” : integerValue1, “SequenceID” : integerValue2} \r\n |
| The following items are optional: “ACC” : byteValue2, “OffsetPRNumber” : shortValue2, “VisionPRNumber” : shortValue3, “MROT” : “ON”, “LCBType” : stringValue3, “LCBValue” : shortValue4, “PortType” : byteValue3, “portNumber” : shortValue5, “portValue” : stringValue4, “ToolOffsetPRNumber” : shortValue6, “NoBlend” : “ON” } \r\n |  |

This packet adds a joint motion instruction to the RMI_MOVE TP program. The only difference between this packet and the FRC_JointMotion packet is the new instruction has joint angle representation instead of Cartesian representation. Without any motion option, a basic FRC_JointMotionJRep packet creates the following TP motion line:

J P[10] 2% FINE

After the controller executes this joint motion instruction, the controller sends the return packet back. The return value integerValue1 = 0 means that the motion instruction successfully executed. If integerValue1 > 0, there is an error in executing the motion and the integerValue1 is the error code.

B-84184EN/03

2 REMOTE MOTION APPLICATION

PROGRAM INTERFACE

NOTE

If your controller does not have extended axes, you can ignore the J7, J8 and J9 keys in the JointAngle.

#### Packet to add two groups joint motion with joint representation instruction

| Remote Device Instruction Packet | Robot Controller Return Packet |
| --- | --- |
| {“Instruction” : “FRC_JointMotionJRep”, “SequenceID” : integerValue, “G1” : { “JointAngle” : { “J1” : floatJ1, “J2” : floatJ2, “J3” : floatJ3, “J4” : floatJ4, “J5” : floatJ5, “J6” : floatJ6, “J7” : floatJ7, “J8” : floatJ8, “J9” : floatJ9} }, “G2” : { “JointAngle” : { “J1” : floatJ1, “J2” : floatJ2, “J3” : floatJ3, “J4” : floatJ4, “J5” : floatJ5, “J6” : floatJ6, “J7” : floatJ7, “J8” : floatJ8, “J9” : floatJ9} }, “SpeedType” : stringValue1, “Speed” : shortValue1, “TermType” : stringValue2, “TermValue” : byteValue1, | {“Instruction” : “FRC_JointMotionJRep”, “ErrorID” : integerValue1, “SequenceID” : integerValue2} \r\n |
| The following items are optional: “ACC” : byteValue2, “OffsetPRNumber” : shortValue2, “VisionPRNumber” : shortValue3, “MROT” : stringValue3: “ON” or “OFF”, “LCBType” : stringValue3, “LCBValue” : shortValue4, “PortType” : byteValue3, “portNumber” : shortValue5, “portValue” : stringValue5, “ToolOffsetPRNumber” : shortValue6, “NoBlend” : “ON” } \r\n |  |

This protocol is the same as FRC_JointMotion and only the difference is that the first position must be in joint angles. Please refer to section Section 2.4.9, Packet To Add Joint Motion Instruction FRC_JointMotion for detail description. Note that joint motion does not support the COORD motion

2 REMOTE MOTION APPLICATION

PROGRAM INTERFACE	B-84184EN/03

option. If you specified COORD in a joint motion instruction, RMI will return an RMIT-041 Joint motion with COORD error.

    - Packet To Add Joint Incremental Motion With Joint Representation

| Remote Device Instruction Packet | Robot Controller Return Packet |
| --- | --- |
| {“Instruction” : “FRC_JointRelativeJRep”, “SequenceID” : integerValue, “JointAngle” : { “J1” : floatJ1, “J2” : floatJ2, “J3” : floatJ3, “J4” : floatJ4, “J5” : floatJ5, “J6” : floatJ6, “J7” : floatJ7, “J8” : floatJ8, “J9” : floatJ9 }, “SpeedType” : stringValue1, “Speed” : shortValue1, “TermType” : stringValue2, “TermValue” : byteValue1, | {“Instruction” : “FRC_JointRelativeJRep”, “ErrorID” : integerValue1, “SequenceID” : integerValue2} \r\n |
| The following items are optional: “ACC” : byteValue2, “OffsetPRNumber” : shortValue2, “VisionPRNumber” : shortValue3, “MROT” : “ON”, “LCBType” : stringValue3, “LCBValue” : shortValue4, “PortType” : byteValue3, “portNumber” : shortValue5, “portValue” : stringValue4, “ToolOffsetPRNumber” : shortValue6, “NoBlend” : “ON” } \r\n |  |

This packet adds an incremental joint motion instruction to the RMI_MOVE TP program. The only difference between this packet and the FRC_JointRelative packet is that the new instruction has joint angle representation instead of Cartesian representation. Without any motion option, a basic FRC_JointRelativeJRep packet creates the following TP motion line:

J P[10] 2% FINE INC

After the controller executes this incremental joint motion instruction, the controller sends the return packet back. If the return value integerValue1 = 0 it means that the motion instruction was

B-84184EN/03

2 REMOTE MOTION APPLICATION

PROGRAM INTERFACE

successfully executed. If integerValue1 > 0, there was an error in executing the motion and the

integerValue1 is the error code.

NOTE

If your controller does not have extended axes, you can ignore the “J7”, “J8” and “J9” keys in the JointAngle.

#### Packet to add two groups joint incremental motion with joint representation instruction

The two groups FRC_JointRelativeJRep instruction shares the same protocol as the two groups FRC_JointMotionJRep instruction defined in Section 2.4.13.1, Packet to add two groups joint motion with joint representation instruction. Please use the same protocol defined in Section 2.4.13.1, Packet to add two groups joint motion with joint representation instruction, change the value of the Instruction key from FRC_JointMotionJRep to FRC_JointRelativeJRep , replace the absolute joint angles with incremental joint angles and you have the two groups joint incremental motion with joint representation packet for RMI.

    - Packet To Add Linear Motion With Joint Representation

| Remote Device Instruction Packet | Robot Controller Return Packet |
| --- | --- |
| {“Instruction” : “FRC_LinearMotionJRep”, “SequenceID” : integerValue, “JointAngle” : { “J1” : floatJ1, “J2” : floatJ2, “J3” : floatJ3, “J4” : floatJ4, “J5” : floatJ5, “J6” : floatJ6, “J7” : floatJ7, “J8” : floatJ8, “J9” : floatJ9 }, “SpeedType” : stringValue1, “Speed” : shortValue1, “TermType” : stringValue2, “TermValue” : byteValue1, | {“Instruction” : “FRC_LinearMotionJRep”, “ErrorID” : integerValue1, “SequenceID” : integerValue2 } \r\n |

2 REMOTE MOTION APPLICATION

PROGRAM INTERFACE	B-84184EN/03

| Remote Device Instruction Packet | Robot Controller Return Packet |
| --- | --- |
| The following items are optional: “ACC” : byteValue2 “OffsetPRNumber” : shortValue2, “VisionPRNumber” : shortValue3, “WristJoint” : “ON”, “MROT” : “ON”, “LCBType” : stringValue3, “LCBValue” : shortValue4, “PortType” : byteValue3, “portNumber” : shortValue5, “portValue” : stringValue4, “ToolOffsetPRNumber” : shortValue6, “ALIM”:integerValue1, “ALIMREG”:shortValue7, “NoBlend” : “ON” } \r\n |  |

This packet adds a linear motion instruction to the RMI_MOVE TP program. The only difference between this packet and the FRC_LinearMotion packet is that the new instruction has joint angle representation instead of Cartesian representation. Without any motion option, a basic FRC_LinearMotionJRep packet creates a TP motion line as shown below. Assuming the stringValue1 = “mmsec”, shortValue1 = 150 with “CNT” termination type and byteValue1 = 100, then the following instruction is added to the TP program:

L P[10] 150mm/sec CNT100

After the controller executes this instruction, the controller sends the return packet back. If integerValue1 = 0 it means that the motion instruction was successfully executed. If integerValue1 > 0, it means that there was an error in executing the motion and the integerValue1 is the error code.

B-84184EN/03

2 REMOTE MOTION APPLICATION

PROGRAM INTERFACE

#### Packet to add two groups linear motion with joint representation instruction

| Remote Device Instruction Packet | Robot Controller Return Packet |
| --- | --- |
| {“Instruction” : “FRC_JointMotionJRep”, “SID” : integerValue, “G1” : { “JointAngle” : { “J1” : floatJ1, “J2” : floatJ2, “J3” : floatJ3, “J4” : floatJ4, “J5” : floatJ5, “J6” : floatJ6, “J7” : floatJ7, “J8” : floatJ8, “J9” : floatJ9 } }, “G2” : { “JointAngle” : { “J1” : floatJ1, “J2” : floatJ2, “J3” : floatJ3,“J4” : floatJ4, “J5” : floatJ5, “J6” : floatJ6, “J7” : floatJ7, “J8” : floatJ8, “J9” : floatJ9 } }, “SpeedType” : stringValue1, “Speed” : shortValue1, “TermType” : stringValue2, “TermValue” : byteValue1, | {“Instruction” : “FRC_JointMotionJRep”, “ErrorID” : integerValue1, “SequenceID” : integerValue2} \r\n |
| The following items are optional: “ACC” : byteValue2, “OffsetPRNumber” : shortValue2, “VisionPRNumber” : shortValue3, “WristJoint” : stringValue3: “ON” or “OFF”, “MROT” : stringValue4: “ON” or “OFF”, “COORD” : stringValue5: “ON” or “OFF”, “LCBType” : stringValue6, “LCBValue” : shortValue4, “PortType” : byteValue3, “portNumber” : shortValue5, “portValue” : stringValue7, “ToolOffsetPRNumber” : shortValue6, “NoBlend”:“ON”} \r\n |  |

2 REMOTE MOTION APPLICATION

PROGRAM INTERFACE	B-84184EN/03

    - Packet To Add Linear Incremental Motion With Joint Representation

| Remote Device Instruction Packet | Robot Controller Return Packet |
| --- | --- |
| {“Instruction” : “FRC_LinearRelativeJRep”, “SequenceID” : integerValue, “JointAngle” : { “J1” : floatJ1, “J2” : floatJ2, “J3” : floatJ3, “J4” : floatJ4, “J5” : floatJ5, “J6” : floatJ6, “J7” : floatJ7, “J8” : floatJ8, “J9” : floatJ9 }, “SpeedType” : stringValue1, “Speed” : shortValue1, “TermType” : stringValue2, “TermValue” : byteValue1, | {“Instruction” : “FRC_LinearRelativeJRep”, “ErrorID” : integerValue1, “SequenceID” : integerValue2 } \r\n |
| The following items are optional: “ACC” : byteValue2 “OffsetPRNumber” : shortValue2, “VisionPRNumber” : shortValue3, “WristJoint” : “ON”, “MROT” : “ON”, “LCBType” : stringValue3, “LCBValue” : shortValue4, “PortType” : byteValue3, “portNumber” : shortValue5, “portValue” : stringValue4, “ToolOffsetPRNumber” : shortValue6, “ALIM”:integerValue1, “ALIMREG”:shortValue7, “NoBlend” : “ON” } \r\n |  |

The FRC_LinearRelativeJRep packet is the same as FRC_LinearRelative, except that the position data is in joint representation. For a simple FRC_LinearRelativeJRep packet, the controller creates a TP line as follows:

L P[10] 150mm/sec CNT100 INC

B-84184EN/03

2 REMOTE MOTION APPLICATION

PROGRAM INTERFACE

After the controller executes this incremental linear motion instruction, the controller sends the return packet back. If integerValue1 = 0 it means that the motion instruction successfully executed. If integerValue1 > 0, there was an error in executing the motion and the integerValue1 is the error code.

#### Packet to add two groups linear incremental motion with joint representation instruction

The two groups FRC_LinearRelativeJRep instruction shares the same protocol as the two groups FRC_LinearMotionJRep instruction defined in Section 2.4.15.1, Packet to add two groups linear motion with joint representation instruction. Please use the same protocol defined in Section 2.4.15.1, Packet to add two groups linear motion with joint representation instruction, change the value of the Instruction key from FRC_LinearMotionJRep to FRC_LinearRelativeJRep, replace the absolute Cartesian position with incremental position and you have the two groups linear incremental motion with joint representation packet for RMI.

NOTE

R904 Spline Motion is required for using this packet

Packet To Add Spline Motion Instruction

2 REMOTE MOTION APPLICATION

PROGRAM INTERFACE	B-84184EN/03

| Remote Device Instruction Packet | Robot Controller Return Packet |
| --- | --- |
| {“Instruction”: “FRC_SplineMotion”, “SequenceID” : integerValue, “Configuration” : { “UToolNumber” : byteValue1, “UFrameNumber”: byteValue2, “Front”: byteValue3,“Up” : byteValue4, “Left” : byteValue5, “Flip” : byteValue6, “Turn4” : byteValue7, “Turn5” : byteValue8, “Turn6”: byteValue9}, “ Position” : { “X”: floatX,“Y” : floatY,“Z” : floatZ, “W”: floatW,“P” : floatP,“R” : floatR, “Ext1” : floatE1, “Ext2” : floatE2, “Ext3” : floatE3}, “SpeedType”: stringValue1, “Speed” : shortValue1, “TermType” : stringValue2, “TermValue” : byteValue10, | {“Instruction” : “FRC_SplineMotion”, “ErrorID” : integerValue1, “SequenceID”: integerValue2}\r\n |
| The following are optional: “ACC” : byteValue11, “OffsetPRNumber” : shortValue2, “VisionPRNumber”: shortValue3, “LCBType” : stringValue3, “LCBValue” : shortValue4, “PortType” : byteValue12, “portNumber”: shortValue5, “portValue” : stringValue4, “ToolOffsetPRNumber” : shortValue6} \r\n |  |

This packet adds a spline motion instruction to the running TP program:

S P[19] 200mm/sec CNT100

The spline motion supports similar motion options as the linear motion. Please refer to the linear motion instruction for a detailed description.

The spline motion requires at least one more motion instruction for the motion system to plane the spline move. Therefore, if RMI receives one spline instruction, the TP program will not execute this instruction until RMI receives one or more motion instructions. RMI leaves the spline motion mode when it receives

B-84184EN/03

2 REMOTE MOTION APPLICATION

PROGRAM INTERFACE

a non-spline motion instruction. Note: when RMI is in spline motion mode, FRC_CALL packet is not allowed and the external device will receive RMIT-056 Need to finish S-motion error in return.

After the controller executes this spline motion instruction, the controller sends the return packet back. The return value integerValue1= 0 means that the motion instruction was successfully executed. If integerValue1> 0, there is an error in executing the motion and the integerValue1 is the error code.

NOTE

If your controller does not have extended axes, you can ignore the Ext1, Ext2 and

Ext3 keys in the Position.

2 REMOTE MOTION APPLICATION

PROGRAM INTERFACE	B-84184EN/03

#### Packet to add two groups spline motion instruction

| Remote Device Instruction Packet | Robot Controller Return Packet |
| --- | --- |
| {“Instruction”: “FRC_SplineMotion”, “SequenceID” : integerValue, “G1” : { “Configuration” : { “UToolNumber” : byteValue1, “UFrameNumber”: byteValue2, “Front”: byteValue3,“Up” : byteValue4, “Left” : byteValue5, “Flip” : byteValue6, “T4” : byteValue7, “T5” : byteValue8, “Turn6” : byteValue9}, “Position”: { “X” : floatX1,“Y” : floatY1, “Z”: floatZ1,“W” : floatW1,“P” : floatP1, “R”: floatR1,“Ext1” : floatE1,“Ext2” : floatE2, “Ext3” : floatE3}}, “G2” : { “Configuration” : { “UToolNumber” : byteValue10, “UFrameNumber”: byteValue11, “Front”: byteValue12,“Up” : byteValue13, “Left” : byteValue14, “Flip” : byteValue15, “T4” : byteValue16, “T5” : byteValue17, “Turn6” : byteValue18}, “Position”: { “X” : floatX3,“Y” : floatY3, “Z”: floatZ3,“W” : floatW3,“P” : floatP3, “R”: floatR3,“Ext1” : floatE7, “Ext2” :floatE8, “Ext3” : floatE9}}, “SpeedType” : stringValue1, “Speed” : shortValue1, “TermType” : stringValue2, “TermValue” : byteValue19, | {“Instruction” : “FRC_SplineMotion”, “ErrorID” : integerValue1, “SequenceID”: integerValue2}\r\n |

B-84184EN/03

2 REMOTE MOTION APPLICATION

PROGRAM INTERFACE

| Remote Device Instruction Packet | Robot Controller Return Packet |
| --- | --- |
| The following are optional: “ACC” : byteValue20, “OffsetPRNumber”: shortValue2, “VisionPRNumber”: shortValue3, “LCBType” : stringValue3, “LCBValue” : shortValue4, “PortType” : byteValue21, “portNumber”: shortValue5, “portValue” : stringValue4, “ToolOffsetPRNumber” : shortValue6} \r\n |  |

The first group position should have the same representation as required by the packet. For example, G1 has to have Cartesian position representation when it is the first position data in the FRC_SplineMotion packet. The second position can be either Cartesian representation or joint representation. RMI does not check the second position’s representation type. If you have position data such that G1 has joint angles and G2 has Cartesian position, you can still use FRC_SplineMotion with G2 be the first position data in the packet.

    - Packet To Add Spline Motion With Joint Representation

| Remote Device Instruction Packet | Robot Controller Return Packet |
| --- | --- |
| {“Instruction” : “FRC_SplineMotionJRep”, “SequenceID” : integerValue, “JointAngle” : { “J1”: floatJ1,“J2” : floatJ2, “J3”: floatJ3,“J4” : floatJ4, “J5”: floatJ5,“J6” : floatJ6, “J7”: floatJ7,“J8” : floatJ8, “J9” : floatJ9 }, “SpeedType”: stringValue1, “Speed” : shortValue1, “TermType” : stringValue2, “TermValue” : byteValue1, | {“Instruction” : “FRC_SplineMotionJRep”, “ErrorID” : integerValue1, “SequenceID”: integerValue2}\r\n |

2 REMOTE MOTION APPLICATION

PROGRAM INTERFACE	B-84184EN/03

| Remote Device Instruction Packet | Robot Controller Return Packet |
| --- | --- |
| The following items are optional: “ACC” : byteValue2 , “OffsetPRNumber”: shortValue2, “VisionPRNumber”: shortValue3, “LCBType” : stringValue3, “LCBValue” : shortValue4, “PortType” : byteValue3, “portNumber”: shortValue5, “portValue” : stringValue4, “ToolOffsetPRNumber” : shortValue6 } \r\n |  |

The FRC_SplineMotionJRep packet is the same as FRC_SplineMotion except that the position data is in joint representation. For a simple FRC_SplineMotionJRep packet, the controller creates a TP line as follows:

S P[20] 200mm/sec CNT100

After the controller executes this spline motion instruction, the controller sends the return packet back. If integerValue1= 0 it means that the motion instruction successfully executed. If integerValue1> 0, there was an error in executing the motion and the integerValue1 is the error code.

B-84184EN/03

2 REMOTE MOTION APPLICATION

PROGRAM INTERFACE

#### Packet to add two groups Spline motion with joint representation instruction

| Remote Device Instruction Packet | Robot Controller Return Packet |
| --- | --- |
| {“Instruction”: “FRC_SplineMotionJRep”, “SID” : integerValue, | {“Instruction”: “FRC_SplineMotionJRep”, “ErrorID” : integerValue1, “SequenceID” : integerValue2} \r\n |
| “G1” : { |  |
| “JointAngle” : { “J1” : floatJ1, J2” : floatJ2, “J3”: floatJ3,“J4” : floatJ4,“J5” : floatJ5, “J6”: floatJ6,“J7” : floatJ7,“J8” : floatJ8, “J9” : floatJ9} |  |
| }, |  |
| “G2” : { |  |
| “JointAngle”: { “J1” : floatJ1, “J2” : floatJ2, “J3” : floatJ3, “J4” : floatJ4, “J5” : floatJ5, “J6”: floatJ6, “J7” : floatJ7, “J8” : floatJ8, “J9” : floatJ9} |  |
| }, |  |
| “SpeedType”: stringValue1, |  |
| “Speed” : shortValue1, |  |
| “TermType” : stringValue2, |  |
| “TermValue” : byteValue1, |  |
| The following items are optional: “ACC” : byteValue2, “OffsetPRNumber”: shortValue2, “VisionPRNumber”: shortValue3, “LCBType” : stringValue3, “LCBValue” : shortValue4, “PortType” : byteValue3, “portNumber”: shortValue5, “portValue” : stringValue4, “ToolOffsetPRNumber” : shortValue6} \r\n |  |

    - Unknown Packet Handling

When the RMI software receives a packet that it cannot interpret, that is, this packet does not have the format specified in the above sections, the RMI software will return a packet with an unknown string error:

2 REMOTE MOTION APPLICATION

PROGRAM INTERFACE	B-84184EN/03

| Remote Device Instruction Packet | Robot Controller Return Packet |
| --- | --- |
| RMI cannot interpret the string | {“Command” : “Unknown”, “ErrorID” : integerValue} \r\n |

Since RMI cannot interpret the incoming string, it returns a special Unknown command packet back to the remote device.

If RMI can partially recognize the incoming instruction packet, it will return the sequence ID as well. The format is shown below.

| Remote Device Instruction Packet | Robot Controller Return Packet |
| --- | --- |
| RMI cannot interpret the string | {“Instruction” : “Unknown”, “SequenceID” : integerValue1 “ErrorID” : integerValue2} \r\n |

You can check the ErrorID and find out which field is incorrect or missing for this instruction. For example, if the ErrorID = 2556959 (RMIT-031), it means the motion speed value is invalid.

NOTE

Please see the Error Code Manual for a description of each of the Remote Motion Interface error codes.

  - POSITION RECORDING

There are two methods of recording position data on the robot controller to use this data in your application. The first method uses position registers and the second method uses the RMI Position Record menu.

    - Using Position Registers

You can record the robot position to position registers without connecting the remote device to the robot controller. After you have taught all of the points in the position registers, you can connect the remote device to the robot controller (FRC_Connect) and use the FRC_ReadPositionRegister packet to transfer all of the recorded position register data to the remote device.

    - Using RMI Position Record Menu

You can also choose RMI Position Record under UTILITIES. To use this menu, you should have the remote device already connected to the robot controller.

B-84184EN/03

2 REMOTE MOTION APPLICATION

PROGRAM INTERFACE

Figure 2.5.2 (a) RMI Position Record

Pressing the Record key, the robot controller sends either one of the following packets to the remote device:

| Robot Controller Sent Packet | Remote Device Return Packet |
| --- | --- |
| { “Command” : “FRC_Record”, “PositionID” : uShortValue, “Representation” : “Cartesian”, “Configuration”: { “UToolNumber” : byteValue1, “UFrameNumber” : byteValue2, “Front”: byteValue3, “Up” : byteValue4, “Left” : byteValue5, “Flip”: byteValue6, “Turn4”: byteValue7, “Turn5”: byteValue8, “Turn6”: byteValue9}, “Position” : {“X” : floatX, “Y” : floatY, “Z” : floatZ, “W”: floatW, “P” : floatP, “R”: floatR, “Ext1” : floatE1, “Ext2” : floatE2, “Ext3” : floatE3}} \r\n | None |

2 REMOTE MOTION APPLICATION

PROGRAM INTERFACE	B-84184EN/03

| Robot Controller Sent Packet | Remote Device Return Packet |
| --- | --- |
| { “Command” : “FRC_Record”, “PositionID” : uShortValue, “Representation” : “Joint”, “JointAngle”: { “J1” : floatJ1, “J2” : floatJ2, “J3” : floatJ3, “J4” : floatJ4, “J5” : floatJ5, “J6” : floatJ6, “J7” : floatJ7, “J8” : floatJ8, “J9” : floatJ9}} \r\n | None |

For a multiple group controller, the menu changes as shown in Figure 2.5.2 (b).

Figure 2.5.2 (b) RMI Position Record for Multiple Groups

When the Record button is pressed, RMI will send the selected group position data back to remote device based on the group mask field. Please see section Section 2.3.1, Packet To Initialize Remote Motion for detail description of the Group Mask.

The Current position display is based on the current jogging group selection. For the example above, the Current Position displays group 2 current position. You can use the FCTN key CHANGE GROUP to toggle the display between different groups.

The Group Mask field is not available on a single group system.

For the above example, pressing the Record key, the robot controller sends the one of the following packets to the remote device:

B-84184EN/03

2 REMOTE MOTION APPLICATION

PROGRAM INTERFACE

| Robot Controller Sent Packet | Remote Device Return Packet |
| --- | --- |
| { “Command” : “FRC_Record”, “PositionID” : uShortValue, “Representation” : “Cartesian”, “G1” : {“Configuration”: { “UToolNumber” : byteValue1, “UFrameNumber” : byteValue2, “Front”: byteValue3, “Up” : byteValue4, “Left” : byteValue5, “Flip”: byteValue6, “Turn4”: byteValue7, “Turn5”: byteValue8, “Turn6”: byteValue9}, “Position” : {“X” : floatX1, “Y” : floatY1, “Z” : floatZ1, “W” : floatW1, “P” : floatP1, “R” : floatR1, “Ext1” : floatE1, “Ext2” : floatE2, “Ext3” : floatE3} }, “G2” : {“Configuration”: { “UToolNumber” : byteValue10, “UFrameNumber” : byteValue11, “Front” : byteValue12, “Up” : byteValue13, “Left” : byteValue14, “Flip”: byteValue15, “Turn4”: byteValue16, “Turn5”: byteValue17, “Turn6”: byteValue18}, “Position” : {“X” : floatX2, “Y” : floatY2, “Z” : floatZ2, “W”: floatW2, “P” : floatP2, “R”: floatR2, “Ext1” : floatE4, “Ext2” : floatE5, “Ext3” : floatE6} } }\r\n | None |

2 REMOTE MOTION APPLICATION

PROGRAM INTERFACE	B-84184EN/03

| Robot Controller Sent Packet | Remote Device Return Packet |
| --- | --- |
| { “Command” : “FRC_Record”, “PositionID” : uShortValue, “Representation” : “Joint”, “G1”: { “JointAngle”: { “J1” : floatJ1, “J2” : floatJ2, “J3” : floatJ3, “J4” : floatJ4, “J5” : floatJ5, “J6” : floatJ6, “J7” : floatJ7, “J8” : floatJ8, “J9” : floatJ9}}, “G2”: { “JointAngle”: { “J1” : floatJ10, “J2” : floatJ11, “J3” : floatJ12, “J4” : floatJ13, “J5” : floatJ14, “J6” : floatJ15, “J7” : floatJ16, “J8” : floatJ17, “J9” : floatJ18}}} \r\n | None |

B-84184EN/03	3 REMOTE DEVICE CONTROL FLOW

- REMOTE DEVICE CONTROL FLOW

  - OVERVIEW OF REMOTE DEVICE CONTROL FLOW

The following flow chart shows a typical Remote Motion session between a remote device and the robot controller.

NOTE

When the remote device sends the FRC_Connect packet, the packet should be sent to a fixed port number (16001) on the controller. When the robot controller sends the returned packet back and the connection is accepted, the return packet will include a new port number. All packets after the FRC_Connect should use the new port number to communicate with the robot’s Remote Motion Interface.

3 REMOTE DEVICE CONTROL FLOW	B-84184EN/03

Figure 3.1 Remote Device Control Flow Chart

B-84184EN/03	3 REMOTE DEVICE CONTROL FLOW

  - INSTRUCTION BUFFER HANDLING

The Remote Motion Interface has a limited buffer for the incoming Teach Pendant instructions. Currently, the limit is set to 8. When the buffer is full, the RMI returns an error if the remote device sends an extra TP instruction. Therefore, when designing the remote device interface with the RMI, it is advised to have a hand shaking mechanism that does the following:

- After the FRC_Initialize packet, send several (up to 8) TP instructions to fill up the RMI instruction buffer.

- After the buffer is full, only send the next TP instruction when a previously sent TP instruction is returned by the RMI.

  - HOLD STATE ERROR HANDLING

The controller goes to the HOLD state when the RMI receives an invalid motion instruction. For example, if the controller does not have R640 (MROT) software loaded and it receives a motion instruction with

the MROT option, the controller will return an error packet back to the remote device and the RMI goes into the HOLD state. In the HOLD state, the controller is still executing the RMI TP program, but it blocks any new instructions to the TP program until it receives a FRC_Reset command. This allows the remote device to react to the error without stopping the execution of the TP program.

Once the remote device corrects the error, it can set FRC_Reset and then continue to send the correct motion instructions. All instructions sent during the HOLD state are ignored and returned back to the sender with an error code.

  - JUMP AND SKIP INSTRUCTIONS

The running RMI_MOVE TP program cannot execute a JUMP or SKIP instruction since this TP program is dynamically updated during its execution. Therefore, you have to manage the SKIP or JUMP instruction in your application program by calling FRC_Abort to terminate the current RMI_MOVE TP program, and use FRC_Initalize to create the program again. When the new TP program is created, you can add the required TP instructions back to the new RMI_MOVE TP program.

  - ERROR HANDLING

There are two type of errors that can happen during the remote motion execution:

- Recoverable: For some warning errors, you may be able to reset the error by sending the FRC_Reset packet. If the return packet shows that the reset is successful, use the FRC_Continue packet to continue the current teach pendent program execution. Note that after sending the FRC_Reset

- REMOTE DEVICE CONTROL FLOW	B-84184EN/03

command, it takes time for the robot controller to reset its error. Please wait for the return of the FRC_Reset packet before sending the FRC_Continue packet. Otherwise, the FRC_Continue command may fail.

  - Non-Recoverable: For any non-warning error, such as a limit error, you have to do the following to recover:

    - Abort the current RMI_MOVE TP program to flush out all of the instructions that are already sent to the controller using the FRC_Abort packet.

    - Go to the controller to clear the error.

    - Send a FRC_Initialize packet to start the RMI_MOVE TP program again following a new set of instructions to continue your process.

  - VISION PROCESS

In order to use FANUC vision, you can run your vision process as a background task. The Remote Motion Interface can send a DOUT (FRC_WriteDOUT) to start the vision process and use FRC_WaitDIN to wait for the vision process to complete before the TP program executes its next instruction. If the vision process is synchronous with the RMI_MOVE program, you can also use FRC_CALL to call the vision process.

B-84184EN/03	4 DATA LOGGING

- DATA LOGGING

In order to facilitate debugging an RMI session, a data log is created to log all the interaction between the controller and the remote device. This log can be viewed on the TP by pressing MENU > NEXT > BROWSER > RMI_LOG.

The Remote Motion Interface creates a data log for all incoming instruction/command packets with a time stamp and some description of the packets.

Figure 4 Example Log File

From Figure 4, you can see the controller receives an FRC_Initialize packet from the remote device at the timestamp 55567. The controller returns the FRC_Intialize at timestamp 55675 and the TP program is initialized successfully.

The remote device immediately sends the FRC_SetUFrame instruction and the controller executes this instruction at the timestamp 55689.

- DATA LOGGING	B-84184EN/03

B-84184EN/03

- ASCII STRING INSTRUCTION

PACKETS

- ASCII STRING INSTRUCTION PACKETS

A Teach Pendent program can be printed out as a LS file using the PRINT function key in the SELECT menu. An LS file can have five sections:

- /PROG (program name)

- /ATTR (program attributes)

- /APPL(Application)

- /MN (program instructions)

- /POS (position data)

For V9.30P/13 (7DF3/13) and later software and V9.40P/08 (7DF5/08) and later software, the remote application can send ASCII strings from the LS file’s /MN section to RMI and RMI will add the equivalent binary TP instruction to the running RMI_MOVE TP program. In the following sections, we will use ASCII string interchangeably as the ASCII string in the /MN section of an LS file.

The new ASCII string support adds lots of new TP instructions to the RMI. However, not all TP instruction are supported due to the sequential execution nature of the RMI TP program. That is, all TP instructions in the RMI_MOVE program are executed sequentially, and any instruction that changes the order of the execution is not allowed. Some of the invalid TP instructions are listed in a later section.

Some other instructions, such as touch sensing search instructions, write data into the TP program when the instruction is executed. Since RMI is writing to the RMI_MOVE TP program, it prevents other software from writing to the same program. Therefore, any instructions that write to the TP program will fail under RMI.

Another issue with the ASCII string support is that some motion instructions, such as ARC motion instruction, require multiple motion lines in the TP Program when the first motion instruction is executed. If there are insufficient ARC motion lines in the TP program when the first ARC motion is executed, the controller stops the program execution with an INTP-609 error. Therefore, when sending ARC motion instructions, users have to make sure the program will not execute the first ARC motion until at least three ARC motion instructions are written into the TP program.

  - ASCII STRING PACKET HANDSHAKING

When loading an LS file into the controller, the LS file goes through an ASCII to binary conversion that converts the ASCII file into a binary Teach Pendant program. RMI uses the same process for processing the ASCII string sent through the RMI connection. Since the conversion process runs outside of the RMI, RMI will have to wait for the conversion process to complete before it can process the next ASCII string packet. To make sure the remote application will wait for the ASCII to binary conversion, the handshaking between the RMI and the RMI application is changed from Figure 5.1 (a) to Figure 5.1 (b).

5 ASCII STRING INSTRUCTION

PACKETS	B-84184EN/03

Figure 5.1 (a) Current RMI and RMI Application Handshaking Sequence

Figure 5.1 (b) RMI with ASCII String and RMI Application Handshaking

  - SYSTEM VARIABLE SETTING

To enable the ASCII string support in RMI, set the system variable $rmi_cfg.$asbn_enb to TRUE and recycle power. Since the ASCII string packet and the JSON based instruction packet have different handshaking, you cannot mix the ASCII string packet with the JSON based instruction packet. By default, this system variable is set to FALSE so RMI is compatible with the current version of the RMI

B-84184EN/03

5 ASCII STRING INSTRUCTION

PACKETS

application that uses JSON based instruction packets. If a user sends an ASCII string packet to RMI when the system variable is set to FALSE, RMI returns RMIT-050 Invalid ASCII String Packet. If the $rmi_cfg.$asbn_enb is set to TRUE and the user sends a JSON instruction packet, RMI returns

RMIT-049 Invalid Instruction Packet.

The ASCII string is very flexible. However, the ASCII string packet goes through the ASCII to binary conversion process so it takes a longer time to convert the ASCII string to a TP instruction. Whereas the RMI converts the JSON instruction to a TP instruction directly, the ASCII conversion requires an additional 4 to 16 milliseconds, depending on the complexity of the ASCII string. If your RMI application is a time critical one, please use the JSON instruction packet instead of the ASCII string packet by setting

$rmi_cfg.$asbn_enb = FALSE.

When using the ASCII string packet, the RMI application needs to consider that it is possible that both the FRC_AsbnReady packet and a return packet will be sent out by RMI around the same time. In this case, the TCP/IP will combine both packets into one TCP/IP packet and the application will only receive one TCP/IP packet instead of two. The RMI Application needs to handle the TCP/IP packet with multiple RMI packets in order to process these two RMI packets individually. It does this by using “\r\n” as a delimiter to separate these RMI packets.

  - ASCII STRING INSTRUCTION PACKET

    - ASCII String Packet For Single Group Controller

| Remote Device ASCII Packet | Robot Controller Return Packet |
| --- | --- |
| {“ASCII” : “ASCII String”, “SequenceID” : integerValue, | {“ASCII” : “ASCII String”, “ErrorID” : integerValue1, “SequenceID” : integerValue2} \r\n |
| See note 1 below explaining Option 1; no position data. |  |

5 ASCII STRING INSTRUCTION

PACKETS	B-84184EN/03

| Remote Device ASCII Packet | Robot Controller Return Packet |
| --- | --- |
| The following items are optional. See note 2 below explaining Option 2. “Configuration” : { “UtoolNumber” : byteValue1, “UFrameNumber” : byteValue2, “Front” : byteValue3, “Up” : byteValue4, “Left” : byteValue5, “Flip” : byteValue6, “Turn4” : byteValue7, “Turn5” : byteValue8, “Turn6” : byteValue9 }, “Position” : { “X” : floatX, “Y” : floatY, “Z” : floatZ, “W” : floatW, “P” : floatP, “R” : floatR, “Ext1” : floatE1, “Ext2” : floatE2, “Ext3” : floatE3} |  |

B-84184EN/03

5 ASCII STRING INSTRUCTION

PACKETS

| Remote Device ASCII Packet | Robot Controller Return Packet |
| --- | --- |
| The following items are optional. See note 3 below explaining Option 3. “Configuration” : { “UtoolNumber” : byteValue1, “UFrameNumber” : byteValue2, “Front” : byteValue3, “Up” : byteValue4, “Left” : byteValue5, “Flip” : byteValue6, “Turn4” : byteValue7, “Turn5” : byteValue8, “Turn6”: byteValue9}, “Position”: { “X” : floatX, “Y” : floatY, “Z” : floatZ, “W” : floatW, “P” : floatP, “R” : floatR , “Ext1” : floatE1, “Ext2” : floatE2, “Ext3” : floatE3}, “Via Configuration”: { “UtoolNumber” : byteValue10, “UFrameNumber” : byteValue11, “Front” : byteValue12, “Up” : “byteValue13” , “Left” : byteValue14, “Flip” : byteValue15 “Turn4” : byteValue16, “Turn5” : byteValue17, “Turn6” : byteValue18 }, “Via Position” : { “X” : floatX, “Y” : floatY, “Z” : floatZ, “W” : floatW, “P” : floatP, “R” : floatR, “Ext1” : floatE4, “Ext2” : floatE5, “Ext3” : floatE6} |  |
| The following items are optional. See note 4 below explaining Option 4. “JointAngle” : { “J1” : floatJ1, “J2” : floatJ2, “J3” : floatJ3, “J4” : floatJ4, “J5” : floatJ5, “J6” : floatJ6, “J7” : floatJ7, “J8”: floatJ8, “J9” : floatJ9} |  |

5 ASCII STRING INSTRUCTION

PACKETS	B-84184EN/03

| Remote Device ASCII Packet | Robot Controller Return Packet |
| --- | --- |
| } \r\n |  |

NOTE

For a single group controller, the ASCII key can have 4 options where option 2, 3 and 4 are position data used in a motion instruction:

1 Option 1: The ASCII instruction does not have a position data associated with this instruction. For example, the packet from the remote device can be {“ASCII” : “PAYLOAD[2] ; “} \r\n

2 Option 2: The ASCII instruction is a motion instruction that includes Cartesian position data.

3 Option 3: the ASCII instruction is a motion instruction that includes a Cartesian via position and destination position data.

4 Option 4: The ASBN instruction is a motion instruction that includes position data in joint representation.

The “ASCII String” has to be a valid LS instruction line in the \MN section of an LS file, including the ‘;’ at the end of the line. The maximum length of the “ASCII String” is 127 bytes.

There are several limitations on the “ASCII string” value:

- The RMI does not preserve the position ID. For example, if the remote device sends the following ASCII string: {“ASCII” : “L P[5] 200 mm/sec FINE ;” …… } \r\n, the RMI will not use 5 in the position ID in the RMI_MOVE TP program since RMI assigns an internal position ID number for each new motion instruction.

- All motion instructions having a position ID should provide position data within the ASCII packet using option 2, 3 or 4. If there is no position data, an error will be posted.

- If the user sends multiple position data with a motion instruction, RMI will return an error.

- If user sends option 3 with linear or joint motion, the via position is ignored.

- If a motion instruction uses a position register, RMI preserves the position register number. However, if the position register is not initialized, a run time error will occur since RMI does not check the validity of the position register before writing the TP instruction to the RMI_MOVE program.

- RMI does not check the validity of motion options. For example, if the “ASCII String” is a motion instruction with the MROT option, RMI does not check whether the MROT motion option is loaded or not before writing the TP binary code to the RMI_MOVE program. If the MROT motion option is not loaded, the execution of MROT will cause a run time program execution error.

- RMI does not check motion option conflicts. For example, an RMI application can sent a multiple group system with coordinated motion option with {“ASCII” : “J P[10] 10% CNT100 COORD ;”,

….} \r\n. RMI will write the TP instruction in the RMI_MOVE program, however since joint motion is illegal for coordinated motion option, this packet will cause a run time program execution error.

- RMI cannot provide any error information for an illegal “ASCII String” string. Instead, the RMI returns ErrorID = 1441794 (ASBN-002). Users have to go to the controller’s alarm page to find out the issue with the ASCII string.

B-84184EN/03

5 ASCII STRING INSTRUCTION

PACKETS

    - ASCII String Packet For Two Group Motion Instruction

RMI can support a TP motion instruction for two groups in a multiple group controller.

| Remote Device ASBN Packet | Robot Controller Return Packet |
| --- | --- |
| {“ASCII” : “ASCII String”, “SequenceID” : integerValue, | {“ASCII” : “ASCII String”, “ErrorID” : integerValue1, “SequenceID” : integerValue2} \r\n |
| The following items are Optional: “G1” : {Option 2 or Option 3 or Option 4}, “G2” : {Option 2 or Option 3 or Option 4} |  |
| } \r\n |  |

NOTE

“G1” and “G2” should be replaced by other valid group numbers as specified in the

FRC_Initialize packet.

RMI does not check whether the ASCII instruction has valid position data or not. If there is not enough position data provided by the remote application, it will cause a run time program execution error.

    - Communication Packet

| Robot Controller ASCII to Binary Ready Packet | Remote Device Return Packet |
| --- | --- |
| {“Communication” : “FRC_AsbnReady” } \r\n | None |

When $rmi_cfg.$asbn_enb = TRUE, RMI will send this packet to the remote device to indicate the ASCII to binary conversion has completed successfully and the TP instruction is written to the RMI_MOVE TP program. Otherwise, RMI will return a packet with a non-zero errorID. For the first eight ASCII String instruction packets, the remote application can send the next ASCII packet to the RMI after it received the FRC_AsbnReady packet. After the instruction buffer is filled, the remote application has to wait for both the FRC_AsbnReady packet and a returned instruction packet before it can send the next ASCII packet.

    - Modified FRC_Initialize Command

To support multiple tools, the FRC_Initialize packet adds two new optional fields as shown in the table below.

5 ASCII STRING INSTRUCTION

PACKETS	B-84184EN/03

| Remote Device Command Packet | Robot Controller Return Packet |
| --- | --- |
| {“Command” : “FRC_Initialize”, The following items are optional: “GroupMask” : unsignedByteValue, “Application” : stringValue1, “Equipment” : byteValue, “RTSA” : stringValue2 } \r\n | {“Command” : “FRC_Initialize”, “ErrorID” : integerValue , “GroupMask”: unsignedByteValue} \r\n |

The StringValue1 has the following valid options:

- “Handling”

- “Arc”

- “Spot”

- “Dispense”

If the Application key is not included in the FRC_Initialize packet, RMI creates the RMI_MOVE program without the application tool specification in its program header. Otherwise, RMI creates the RMI_MOVE TP program header accordingly. However, if the controller’s application tool setting is not compatible with StringValue, RMI will return an error.

RMI also support multiple equipment in the RMI_MOVE program header for ARC and Spot application. The ByteValue is the equipment number for RMI to write to the RMI_MOVE program header. RMI does not check the validity of the equipment number.

NOTE

If the controller is loaded with the handling tool or dispense tool, the equipment number is ignored.

For controller loaded with singularity avoidance option, RMI will automatically disable the singularity avoidance in RMI_MOVE program header. Since the remote RMI application issues the motion instructions, the remote RMI application should plan motion around singularity by itself.

For controller loaded with tracking option, RMI will automatically disable the tracking option in

RMI_MOVE program header since RMI does not support tracking.

For HandlingTool application, by default, the RMI_MOVE program is created as JOB program type. For Spot, Arc and Dispense, the RMI_MOVE program is created as NONE program type.

When $rmi_cfg.$asbn_enb = FALSE, the Application and Equipment keys are ignored.

The RTSA key is an optional key for a controller loaded with the Singularity Avoidance option “R512” or “R513”. When this optional key set to “ON”, RMI enables the Singularity Avoidance feature in the RMI_MOVE.TP program. However, if the Singularity Avoidance option is not loaded, RMI returns ErrorID = 2556980.

B-84184EN/03

5 ASCII STRING INSTRUCTION

PACKETS

  - LIST OF INVALID ASCII STRING INSTRUCTIONS

RMI does not allow instructions that change the order of execution of the RMI_MOVE program. The following is a list of known instructions that RMI does not support and will return an ErrorID = 2556975 (RMIT-047).

- Jump/Label instructions.

- Run instruction.

- IF/Select instruction.

- Skip instruction.

- FOR/ENDFOR instruction.

5 ASCII STRING INSTRUCTION

PACKETS	B-84184EN/03

APPENDIX

B-84184EN/03	A APPENDIX

- APPENDIX

  - RMI ERRORID REFERENCE TABLE

The following table provides a quick reference for the return packet's ErrorID.

| ErrorID | RMI Error | Error Message |
| --- | --- | --- |
| 2556929 | RMIT-001 | Internal System Error. |
| 2556930 | RMIT-002 | Invalid UTool Number. |
| 2556931 | RMIT-003 | Invalid UFrame Number. |
| 2556932 | RMIT-004 | Invalid Position Register. |
| 2556933 | RMIT-005 | Invalid Speed Override. |
| 2556934 | RMIT-006 | Cannot Execute TP program. |
| 2556935 | RMIT-007 | Controller Servo is Off. |
| 2556936 | RMIT-008 | Teach Pendant is Enabled. |
| 2556937 | RMIT-009 | RMI is Not Running. |
| 2556938 | RMIT-010 | TP Program is Not Paused. |
| 2556939 | RMIT-011 | Cannot Resume TP Program. |
| 2556940 | RMIT-012 | Cannot Reset Controller. |
| 2556941 | RMIT-013 | Invalid RMI Command. |
| 2556942 | RMIT-014 | RMI Command Fail. |
| 2556943 | RMIT-015 | Invalid Controller State. |
| 2556944 | RMIT-016 | Please Cycle Power. |
| 2556945 | RMIT-017 | Invalid Payload Schedule. |
| 2556946 | RMIT-018 | Invalid Motion Option. |
| 2556947 | RMIT-019 | Invalid Vision Register. |
| 2556948 | RMIT-020 | Invalid RMI Instruction. |
| 2556949 | RMIT-021 | Invalid Value. |
| 2556950 | RMIT-022 | Invalid Text String |
| 2556951 | RMIT-023 | Invalid Position Data |
| 2556952 | RMIT-024 | RMI is In HOLD State |
| 2556953 | RMIT-025 | Remote Device Disconnected. |
| 2556954 | RMIT-026 | Robot is Already Connected. |
| 2556955 | RMIT-027 | Wait for Command Done. |
| 2556956 | RMIT-028 | Wait for Instruction Done. |
| 2556957 | RMIT-029 | Invalid sequence ID number. |
| 2556958 | RMIT-030 | Invalid Speed Type. |

A APPENDIX	B-84184EN/03

| ErrorID | RMI Error | Error Message |
| --- | --- | --- |
| 2556959 | RMIT-031 | Invalid Speed Value. |
| 2556960 | RMIT-032 | Invalid Term Type. |
| 2556961 | RMIT-033 | Invalid Term Value. |
| 2556962 | RMIT-034 | Invalid LCB Port Type. |
| 2556963 | RMIT-035 | Invalid ACC Value. |
| 2556964 | RMIT-036 | Invalid Destination Position |
| 2556965 | RMIT-037 | Invalid VIA Position. |
| 2556966 | RMIT-038 | Invalid Port Number. |
| 2556967 | RMIT-039 | Invalid Group Number |
| 2556968 | RMIT-040 | Invalid Group Mask |
| 2556969 | RMIT-041 | Joint motion with COORD |
| 2556970 | RMIT-042 | Incremental motn with COORD |
| 2556971 | RMIT-043 | Robot in Single Step Mode |
| 2556972 | RMIT-044 | Invalid Position Data Type |
| 2556973 | RMIT-045 | Not Ready for ASCII Packet |
| 2556974 | RMIT-046 | ASCII Conversion Failed |
| 2556975 | RMIT-047 | Invalid ASCII Instruction |
| 2556976 | RMIT-048 | Invalid Number of Groups |
| 2556977 | RMIT-049 | Invalid Instruction packet |
| 2556978 | RMIT-050 | Invalid ASCII packet |
| 2556979 | RMIT-051 | Invalid ASCII string size |
| 2556980 | RMIT-052 | Invalid Application Tool |
| 2556981 | RMIT-053 | Invalid Call Program Name |
| 2556982 | RMIT-054 | Joint Motion with ALIM |
| 2256983 | RMIT-055 | ALIM option is not loaded |
| 2256984 | RMIT-056 | Need to finish S-motion |
| 2256985 | RMIT-057 | Spline option is not loaded |
| 7004 | MEMO-004 | Specific program is in use |
| 7015 | MEMO-015 | Program already exists |

B-84184EN/03	INDEX

INDEX

##### A

Appendix 87

ASCII STRING INSTRUCTION PACKET 77 ASCII STRING INSTRUCTION PACKETS 75

ASCII String Packet For Single Group Controller 77 ASCII String Packet For Two Group Motion Instruction 81 ASCII STRING PACKET HANDSHAKING 75

##### C

Communication Packet 81

COMMUNICATION PACKETS 5

COMPATIBILITY 2

Controller Disconnect Packet 7

##### D

DATA LOGGING 73

##### E

ERROR HANDLING 71

##### H

HARDWARE AND SOFTWARE REQUIREMENTS 1 HOLD STATE ERROR HANDLING 71

Hot Start is Disabled 2

##### I

INSTRUCTION BUFFER HANDLING 71

Instruction Packet Buffer 2 INTRODUCTION 1

##### J

JUMP AND SKIP INSTRUCTIONS 71

##### L

LIMITATIONS 2

LIST OF INVALID ASCII STRING INSTRUCTIONS 83

##### M

Minimum Motion Duration 3 Modified FRC_Initialize Command 81

##### N

Number of Motion Groups Supported 2

##### O

Original Path Resume Function is Disabled 2

Other Available Communication Options 3

OVERVIEW OF REMOTE DEVICE CONTROL FLOW 69

##### P

Packet Exchange 6

Packet To Abort Remote Motion TP Program 11

Packet To Add Circular Incremental Motion Instruction 48 Packet To Add Circular Motion Instruction 44

Packet To Add Joint Incremental Motion Instruction 42 Packet To Add Joint Incremental Motion With Joint Representation 52

Packet To Add Joint Motion Instruction 38

Packet To Add Joint Motion With Joint Representation 50 Packet To Add Linear Incremental Motion Instruction 36 Packet To Add Linear Incremental Motion With Joint Representation 56

Packet To Add Linear Motion Instruction 28

Packet To Add Linear Motion With Joint Representation 53 Packet To Add Spline Motion Instruction 57

Packet To Add Spline Motion With Joint Representation 61 Packet to add two group joint motion instruction 40

Packet to add two groups circular incremental motion instruction 49

Packet to add two groups circular motion instruction 46

Packet to add two groups joint incremental motion instruction 43 Packet to add two groups joint incremental motion with joint representation instruction 53

Packet to add two groups joint motion with joint representation instruction 51

Packet to add two groups linear incremental motion instruction 37

Packet to add two groups linear incremental motion with joint representation instruction 57

Packet to add two groups linear motion instruction 32

Packet to add two groups linear motion with joint representation instruction 55

Packet to add two groups spline motion instruction 60

Packet to add two groups Spline motion with joint representation instruction 63

Packet To Add Wait Time Instruction 26 Packet To Get Controller Status 14

Packet To Get Current User/Tool Frame Number 20 Packet To Initialize Remote Motion 9

Packet To Pause Remote Motion Program 11 Packet To Read Controller Error 12

Packet To Read Current Robot Cartesian Position 18 Packet To Read Current Robot Joint Angles 19 Packet To Read Current Tool Center Point Speed 23 Packet To Read Input Port 17

Packet To Read Position Register Data 21 Packet To Read User Frame Data 15 Packet To Read User Tool Data 16 Packet To Reset Robot Controller 22

Packet To Resume Remote Motion Program 11 Packet To Set Payload Instruction 27

Packet To Set Speed Override 20

Packet To Set The Current UFrame-UTool Number 13 Packet To Set User Frame Data 15

INDEX	B-84184EN/03

Packet To Set User Frame Instruction 26 Packet To Set User Tool Data 17 Packet To Set User Tool Instruction 26 Packet To Wait for DIN Instruction 25 Packet To Write Digital Output Port 18

Packet To Write Position Register Data 22 Position Recording 64

##### R

Remote Device Control Flow 69

REMOTE MOTION APPLICATION PROGRAM INTERFACE 5

REMOTE MOTION APPLICATION PROGRAM INTERFACE OVERVIEW 5

REMOTE MOTION INTERFACE OVERVIEW 1

replace with title name 27

RMI ERRORID REFERENCE TABLE 87 ROBOT COMMAND PACKETS 8

##### S

System Fault Packet 8

SYSTEM VARIABLE SETTING 76

##### T

TEACH PENDANT PROGRAM INSTRUCTION PACKETS 23

Timeout Terminate Packet 8

##### U

Unknown Packet Handling 63 Using Position Registers 64

Using RMI Position Record Menu 64

##### V

VISION PROCESS 72

B-84184EN/03	REVISION RECORD

REVISION RECORD

| Edition | Date | Contents |
| --- | --- | --- |
| /03 | Dec., 2024 | - Modifications to Teach Pendant Program Instructions section. - Addition of Spline motion packet - Modifications to RMI ERRORID reference table. - Addition of Override return packet. |
| /02 | Oct., 2022 | - Addition of the ALIM (Acceleration Limit) motion modifier function. - Support ASCII string packets - New feature description for FRC_Call instruction and Tool_offset, PR[] motion modifier - Multiple Groups |
| /01 | Mar., 2020 | - First Release |

#### B-84184EN/03

*  B -  8 4 1 8 4 E N /  0 3 *